SEIKA

Studio

Braille Computer

User Manual

Version 1.1

February 2023

Preface

Thank you for purchasing the Seika Studio Braille Display.

The Seika Studio is a portable 40 cells Braille computer with the Microsoft Windows operating system. You can install your prefer screen reader and you can do everything which your laptop can do.

Before using the Seika Studio, please read this user manual.

If you have additional questions about using the Seika Studio, please contact distributors by e-mail, letter, fax, or telephone.

NOTE: Contents and specifications are subject to change without prior notice.

Safety Precautions

For your own safety and to protect the Seika Studio from accidental damage, please read and abide by the following safety precautions.

1. The wall adapter input is 110V - 240V, 50/60Hz, and the output is DC 19V / 5A.

2. Your Seika Studio needs to be handled with care. Make sure to handle the unit in a proper manner. Also, the Seika Studio is very sensitive to dust. Make sure to keep Seika Studio away from dusty environments.

3. Please keep your Seika Studio away from a long time in a closed, high temperature environment, such as inside a car on a hot summer day.

4. Do not disassemble the Seika Studio yourself.

5. Please Note that if you do not use Seika Studio several days or have a long trip, please shut down Windows OS and turn off the power switch.

Table of Contents

1. Getting Started	5

1.1 Equipment Checklist	5

1.2 Device description	5

1.3 Power Switch and Battery	7

1.4	Hardware Specifications	8

2. Basic Functions of the Seika Studio	9

2.0.1 How to Use the Commands	9

2.0.2 General Functions	9

2.1 Computer-mode	10

2.2 Braille display mode	10

2.2.1 USB connect mode	11

2.2.2 Bluetooth mode	11

3. Using the Seika Studio with JAWS	11

3.1 Installing the Seika Patch for JAWS	11

3.2 Configuring JAWS	11

3.3 Navigation keys and commands	12

3.4 Uninstalling the patch for JAWS	13

4. Using the Seika Studio with Window-Eyes	13

4.1 Configuring Window-Eyes	13

4.2 Navigation Keys and Commands	14

5. Using the Seika Studio with NVDA	14

5.1 Pairing Braille Devices	14

5.2 Braille Commands	15

6. Using the Seika Studio Bluetooth with VoiceOver	15

6.1 Using VoiceOver on the iPad and iPhone	15

6.2 Using VoiceOver on a Mac OS X computer	16

6.3 Navigation keys and commands	17

7. Using the Seika Studio with android Talkback	18

8. How to re-install the Windows 64-bit OS on the Seika Studio	20

8.1 How to make a bootable Windows 64-bit OS USB disk	20

8.2 How to re-install a Windows 64-bit OS on your Seika Studio	20

# 1. Getting Started

The Seika Studio is a portable 40 cell Braille display computer with the Microsoft Windows operating system. There are three working modes: computer mode, USB connect mode and Bluetooth connect mode. You can install screen reader you like and do everything which your laptop can do in computer mode (for details, please refer to chapter 2.1). The Seika Studio is also compatible with several popular screen reading software programs that are used with mobile phones, PC’s, and Apple devices via Bluetooth or USB cable mode (for details, please refer to chapter 2.2). If you do not use Seika Studio several days or have a long trip, please shut down Windows OS firstly, and turn off the Seika Studio power.

NOTE: When the Seika Studio Windows OS is working, you can use USB connect mode or Bluetooth connect mode connect with mobile phone or another PC.

NOTE: We strongly suggest that before you switch off the Seika Studio power, please shut down the Windows OS firstly, otherwise the computer will be shut down abnormally, and it maybe lose data.

NOTE: When the computer is turned on or turned off, it will beep four tones Do-Re-Mi-Fa or Fa-Mi-Re-Do.

## 1.1 Equipment Checklist

Carefully unpack your Seika Studio. Save the box and packing materials for future use. Your Seika Studio package contains the following items:

- Seika Studio in its leather carrying case with a strap

- Wall adapter (the output is 19V/5A)

- USB Cable

- User manual

## 1.2 Device description

The Seika Studio total weight is 1.6 kg with dimensions of 12.6 (W) x 7.6 (D) x 2.0 (H) inches.

The Seika Studio can operate from its internal rechargeable battery or from the wall adapter.

The Seika Studio comes in a leather carrying case. To open the case, lift the flap under the front edge and fold it back.

Place the Seika Studio on a flat surface with the Braille display facing you.

On the left side, you’ll find from back to front:

- A DC-IN socket.

- A USB Type-C host port socket under the DC-IN socket, which supports display output and USB data transmission with internal computer.

- A USB Type-C device port socket near the USB Type-A host slot, which support USB data transmission when using Seika Studio as a braille display, and also support USB PD charger.

- Two vertical USB Type-A host slots which are for the USB device, such as USB disk, Mouse and keyboard. The slot far away from you is compatible with USB 3.0; the other one near you is compatible with USB 2.0.

- A slide switch surrounded by a concave rectangular frame. This slide switch is for Seika Studio keyboard selection.

- A Headset socket.

On the top surface of the Seika Studio you’ll find:

- 40 refreshable Braille cells (8 dots)

- Corresponding to the Braille cells, there are raised camber ridges through the rear of each cell. These buttons are called the cursor routing keys. They are especially useful when editing a document. In Notepad, for example, when you press a touch cursor button, the cursor moves to that cell. You can then edit the character displayed on that cell. The touch cursor can also be used to select an item from a menu or list.

- Behind the Braille cells are the keyboard. Middle position of the slide switch is QWERTY keyboard selection; this keyboard is working like normal laptop keyboard. Near your side position of slide switch, only ASDFJKL; and space working as the eight braille dots input key, in this 8-key Braille keyboard, you should place your fingers on these 9 keys in the following manner: Place the index finger, middle finger, ring finger, and pinky finger of your left hand on the “ASDF” buttons. Then, place the index finger, middle finger, ring finger, and pinky finger of your right hand on the “JKL;” buttons. Finally, place both of your thumbs on the spacebar. The button where you placed your left index finger corresponds with dot 1 in Braille, the button that your left middle finger is on corresponds to dot 2, and button under your left ring finger corresponds to dot 3. The button on which your left pinky finger is placed is the Backspace key (key 7). The button under your right index finger corresponds with dot 4 in Braille, the button under your right middle finger is dot 5, and the button on which your ring finger is placed corresponds with dot 6 in Braille. The button on which your right pinky is placed is the Enter key (key 8). And, the two buttons under your thumbs are the spacebar.

- On each side of the Braille display is a rectangle shaped key. The left button (LB) is named also K1, and the right button (RB) is also named K8.

- In front of K1 and K8, there are two five-direction (left, right, up, bottom and center) navigation joysticks. The left joystick (LJ) is also named as K2, and the right joystick (RJ) is also named as K7.

- There is a microphone and LED in the center of the same row as the K2 and K7.

On the front side, from left to right, you’ll find:

Four keys, which is named as K3, K4, K5 and K6.

On the behind side, from left to right, you’ll find:

Left Speaker, Windows Computer Button, HDMI slot, Power Switch and right Speaker.

## 1.3 Power Switch and Battery

As you move your hand along the behind side of the case, you will find a power switch surrounded by a concave rectangular frame. This power switch controls the whole power supply of the Seika Studio. Turn on the internal computer needs power on the Studio first.

Move Power Switch to the right side will power on the Studio. There will two beeps and status information Braille display prompt. The prompt includes: battery level (%), charge status (charging/no charge/fully charged), internal computer on/off, keyboard status (lock/qwerty/asdf), Braille display up stream (self/USB/BT) and firmware version. The internal computer will not turn on after power on the Studio for safety, please use the Computer Button to turn on or off the internal computer.

Turn on internal computer by pressing the Computer Button, beeps tone Do-Re-Mi-Fa after press the Computer Button and there will have cycling Braille display prompt after “% on”, “on” means the internal computer is power on. If the Screen Reader can’t start or can’t output Braille in 1 minute, the Braille display will prompt status information instead of cycling. The internal computer not allowed turn on if batter percent less than 10%, the beeps tone “Mi-Re-Do” will prompt after press Computer Button. After turn off the internal computer there has beeps tone Fa-Mi-Re-Do, this means you can switch off the Studio safely.

Please Note that if you do not use Seika Studio several days or have a long trip, please push the switch to the left side which is towards to the HDIM, which is completely power off the Seika Studio.

NOTE: We strongly suggest that before you switch off; please turn off the Windows OS firstly. Otherwise the Windows OS will improperly shut down, it maybe loses your data and it will take more time when you start Windows OS next time.

Seika Studio’s Braille display will enter sleep mode after 1 minute without active. Pressing any key will end the sleep mode, but it is recommended that you momentarily press the Joystick left or Joystick right. This will avoid accidentally inserting characters in an open document.

Seika Studio can operate from its internal rechargeable battery or from its wall adapter. The length of time you can run Seika Studio from a fully charged battery is typically over 8 hours before it becomes flat in computer working mode and over 60 hours in Braille display working mode (USB or Bluetooth connect mode). Activities like using the Wi-Fi, USB port or Bluetooth will increase the drain on the battery.

The USB-C port can be used as data translation and USB PD charge. Seika Studio only support more than 20V USB PD charging. If USB-C connects with PC/Mac, which is only 5V output and this will not charge the Seika Studio.

When the battery level falls below 15%, the Seika Studio displays the alarm message every ten minutes, “xx% lower power please charge” and beeps tone “Mi-Re-Do” suggests you connect the wall adapter, and you can press any key or wait 5 seconds to escape from the alarm message. The DC IN socket is located on the left side, behind the USB Type-C socket. When you connect the wall adapter successfully, it will display “xx % charging”, and you will hear beep tone “Do” which indicates the battery is charging. You can press any key or wait 5 seconds to escape from this message. When the battery level falls below 3%, the Seika Studio will shut down automatically, there beeps tone “Mi-Re-Do”, if the internal computer is running now the Braille display will cycle prompt after “off” meaning for shut down now, after beeps long tone “Fa-Mi-Re-Do” the Studio power off.

If you need to use the unit during the initial recharge, the unit should be connected to the wall adapter for at least five hours in order to fully charge the battery. Charging takes less time if the battery is only partially discharged. You can charge the battery and use the Seika Studio simultaneously although the battery may take longer to charge. Please ensure that the battery charges correctly, you can only charge in ambient temperatures between 41 and 95 degrees Fahrenheit, or 5 to 35 degrees Celsius.

## Hardware Specifications

- Operating system: Windows 10/11-64bit

- Storage: 500GB (up to 2TB) NVMe™ PCIe Gen4 M.2 SSD

- RAM: 8GB (up to 64GB) DDR4

- CPU: 11th Intel® Core™ i3

- Battery: Lithium rechargeable battery

- Keyboard: Laptop keyboard, 6 scroll keys, 40 cursor routing keys, 2 navigation joysticks keys

- Braille display: 40 refreshable 8-Dot Braille cells

- Wireless: WIFI and Bluetooth

- Voice recording: Internal microphone

- Sound: Internal stereo speakers, stereo headset jack

- Video output: HDMI output and USB Type-C Display output

- Interface: USB Type-C host, USB Type-A host (USB 2.0 and USB 3.0), DC IN, USB Type-C device socket

# 2. Basic Functions of the Seika Studio

### 2.0.1 How to Use the Commands

All of the commands used on the Seika Studio are comprised of different combinations of the Braille keys. In this user manual, the “+” sign is used to indicate key combinations. For example, “Space+z” indicates that the user should press the Space key and letter z Braille (dots 1-3-5-6) simultaneously and then release them at the same time. You can execute most commands by pressing the “Enter” button (press Joystick_Center, or press Dot8). If you would like to cancel the execution of any command, or return to the previous status, press “Space +e” (1-5 + Space).

### 2.0.2 General Functions

Upon switching on the unit by using the power switch, it will beep twice, and there will status information prompt on the Braille display. The prompt includes: battery level (%), charge status (charging/no charge/fully charged), internal computer on/off, keyboard status (lock/qwerty/asdf), Braille display source (self/USB/BT) and firmware version.

When you press the windows Computer Button in the behind, then beeps Do-Re-Mi-Fa and Windows OS will start.

NOTE: We strongly suggest that before you switch off, please shut down the Windows OS, otherwise the Windows OS will Non normal shut down, then it maybe lose data and it will take more time when you start Windows OS next time.

NOTE: Please switch off the Studio for long trip or do not use for days.

You can use K1+K3+K7_Bottom, with one beep, to get status information of the Seika Studio. The information format is: battery level (%), charge status (charging/no charge/fully charged), internal computer on/off, keyboard status (lock/qwerty/asdf), Braille display source (self/USB/BT) and firmware version. For example, the Seika Studio status will display: 60% no charge on qwerty self v1, which means the battery level is 60%, no charge, the internal computer is working, keyboard is qwerty which is not use asdfjkl; for emulate 8-key Braille keyboard, the Braille display uses the internal computer as upstream source, and the firmware version is v1. The above information will be displayed 5 seconds, and then it will disappear.

Braille and Hot Key Summary

| Action | Hot key | Braille |
| --- | --- | --- |
| Get the status information | K1+K3+K7_Bottom |  |
| Computer-mode | Left Button + Routing 1 |  |
| USB connect mode | Left Button + Routing 2 |  |
| Bluetooth connect mode | Left Button + Routing 3 |  |
| Lock keyboard | Far away position of slide switch |  |
| QWERTY keyboard | Middle position of slide switch |  |
| 8-key Braille keyboard (asdfjkl; and space) | Near your side position of slide switch |  |
| Turn on/off Internal Computer | Computer Button |  |
| Power on whole device | Move Power Switch to right |  |
| Power off whole device | Move Power Switch to left |  |

## 2.1 Computer-mode

The Computer-mode allows you to use Seika Studio as a laptop with Braille display. You can install your own prefer screen reader for Microsoft Windows. Please refer to chapter 3 to 7 for screen reader which can working with Seika Studio.

Upon switching on by use power switch, it will beep twice to show that the Seika Studio will be working in computer-mode by default. By using Left Button + Routing 1, with beep tone Do, can select the computer-mode manually. When you press the Windows computer button in the behind, then Windows OS will start with starting notice beep, and the Braille display will prompt that the computer is on. It will be taking about 30-50 seconds, and then the Windows OS will reach the welcome message and music.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

If you shut down the windows OS, it will play shut down music and it will be take about 20-30 seconds with beeps notice the system shut down.

NOTE: We suggest you use the Sleep, Hibernate of the Windows OS for daily use of the Seika Studio, and the Seika Studio will speed up the start time and have long battery life.

NOTE: We strongly suggest that before you turn off the power switch, please shut down the Windows OS, otherwise the Windows OS will Non normal shut down, then it maybe lose data and it will take more time when you start Windows OS next time.

NOTE: If you re-install Windows OS, you need install the hardware driver for Seika Studio inside computer. Refer to the chapter 8 for the detail about re-install the Windows OS.

## 2.2 Braille display mode

This will enable you to use your Seika Studio as a Braille display with QWERTY or 8-key input keyboard. Use the Left Button + Routing 2 to select USB connection, use the Left Button + routing 3 to select the Bluetooth connection. The Studio will beep tone Do-Do and Do-Do-Do respectively after select USB and Bluetooth connection mode and Braille display prompts when you select the connections.

### 2.2.1 USB connect mode

This will enable you to use your Seika Studio as a Braille display with QWERTY or 8-key input keyboard. You need connect the USB cable and launch the screen reader on the PC or MAC. The Seika Studio supports most of screen reader software, such as JAWS, Window-Eyes, NVDA, and VoiceOver and so on.

By using the Left Button + Routing 2 to select USB connect mode, the Seika Studio will beep tone Do-Do and display “wait usb connect…”. Then you can start your screen reader software.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

### 2.2.2 Bluetooth mode

This allows you to set up the Bluetooth connection between the Seika Studio and your PC, MAC or cell phone.

By using the Left Button + Routing 3 to select Bluetooth mode, the Seika Studio will beep tone Do-Do-Do and display “wait bt connect...” two seconds later, it will display “tsm **** pin 0000”, where **** is the Bluetooth serial number of Seika Studio, PIN 0000 means when you are asked to input the Seika Studio Bluetooth pin code, enter “0000” to set up the connection.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

# 3. Using the Seika Studio with JAWS

## 3.1 Installing the Seika Patch for JAWS

Open the “Patch for JAWS” folder, and run the Patch of JAWS.

Install the patch by using the default steps. The Patch for JAWS will be installed by the JAWS Utility automatically. A console window appears which will take several minutes.

## 3.2 Configuring JAWS

Set the Seika Studio to USB connect or Bluetooth connect mode by using Left Button + Routing 2 or Left Button + Routing 3 respectively. Please refer to Section 2.2 for more connection steps.

Start JAWS and select “Braille…” in the “Options” menu.

Assuming that the default Braille display is set to “No display”, tab to the “Add Braille Display” button and press Enter.

In the “Select components” dialog box, there is a list of Braille displays. Select the “Seika” by pressing the Space bar.

Tab to the “Next” button and press Enter. If you connected the Seika Studio to your computer by USB cable, select “USB” as the output port.

When prompted to set the primary device, select “Seika” in the “Primary device” list.

Tab to the “Finish” button and press Enter.

In the “Braille Basic Settings” dialog box, tab to the “OK” button and press it.

Restart JAWS and the Seika Studio will work now.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

## 3.3 Navigation keys and commands

| Function | Keys |
| --- | --- |
| Enter | RJ_Center |
| Up Arrow Key | RJ_Up |
| Down Arrow Key | RJ_Bottom |
| Left Arrow Key | RJ_Left |
| Right Arrow Key | RJ_Right |
| Braille Pan Right | RB |
| Braille Pan Left | LB |
| Braille Moves Active | LJ_Left+RJ_Up |
| Active Moves Braille | LJ_Left+RJ_Bottom |
| Braille Toggle Marking | LJ_Center+RJ_Up |
| Braille Next Line | LJ_Right |
| Braille Prior Line | LJ_Left |
| Start JAWS Task List | LJ_Right+RJ_Up |
| List Task Tray Icons | LJ_Right+RJ_Bottom |
| Right Mouse Button | LJ_Right+RJ_Right |
| Left Mouse Button | LJ_Right+RJ_Left |
| Left Mouse Button Lock | A+LJ_Right |
| Route JAWS Cursor To Pc | LJ_Right+RJ_Center |
| Route PC Cursor To JAWS | LJ_Left+RJ_Center |
| Braille Bottom | LJ_Bottom |
| Braille Top | LJ_Up |
| Select A Link | LJ_Bottom+RJ_Left |
| Select A Frame | LJ_Up+RJ_Bottom |
| Select A Heading | LJ_Up+RJ_Left |
| Select A Form Field | LJ_Up+RJ_Right |
| Bottom Of File | LJ_Up+RJ_Up |
| Top of File | LJ_Bottom+RJ_Bottom |
| PC Cursor | RB+LJ_Right |
| JAWS Cursor | RB+LJ_Left |
| Braille Toggle Mode | LB+RB |
| Braille Toggle Characters And Attributes | LJ_Left+RJ_Left |
| Increase Braille Auto Advance Speed | LB+RJ_Center |
| Decrease Braille Auto Advance Speed | LB+LJ_Center |
| JAWS Page Down | LJ_Bottom+RJ_Center |
| JAWS Page Up | LJ_Up+RJ_Center |
| Braille Tab | LJ_Center+RJ_Right |
| Braille Shift Tab | LJ_Center+RJ_Left |
| Next Document Window | RB+LJ_Bottom |
| Previous Document Window | RB+LJ_Up |
| Route Braille To Active Cursor | RB+LJ_Center |
| Braille Select Text | RJ_Center+Routing |
| Repeat Last Flash Message | LJ_Bottom+RJ_Up |
| Braille Auto Advance | LJ_Bottom+RJ_Right |

## 3.4 Uninstalling the patch for JAWS

Note: If you uninstall the JAWS patch, it will disable the Seika Studio’s ability to work with JAWS and delete all the associated patch files. You must uninstall this patch first before uninstalling JAWS.

Open the “Add or Remove Programs” in the “Control Panel”, and select the item: “Seika Patch for JAWS V2.3”. Uninstall the “Seika Patch for JAWS V2.3” step by step. The installation will take several minutes. Please restart the computer after the uninstall finishes.

# 4. Using the Seika Studio with Window-Eyes

Note: We strongly recommend you update Window-Eyes to version 7.5.2 or later.

## 4.1 Configuring Window-Eyes

Set the Seika Studio to USB connect or Bluetooth connect mode by using Left Button + Routing 2 or Left Button + Routing 3 respectively. Please refer to Section 2.2 for more connection steps.

Start Window-Eyes and select “Device” in the “Settings” list.

Select “Braille Display” in the “Device” list.

In the “Braille Displays” option, select “Seika”.

In the “Ports” option, select the USB connection.

Tab to the “Activate” button and press Enter.

Select “Keep Changes” button in the “Braille Settings” dialog box, then press Enter.

Tab to “Save Settings” button and press Enter.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

## 4.2 Navigation Keys and Commands

| Function | Keys |
| --- | --- |
| Bottom Line | RJ_Right |
| Mouse Bottom Left | LB+RJ_Bottom |
| Mouse Bottom Right | RB+LJ_Right |
| Mouse Top Left | LB+RJ_Up |
| Mouse Top Right | RB+LJ_Left |
| To Focus | LB+RB |
| 6/8 Dot Toggle | Space +LB |
| Untranslatable at Cursor | LJ_Center+RJ_Left |
| Untranslatable at Mouse | LJ_Center+RJ_Right |
| Braille Graphic Mode | LJ_Center+RJ_ Bottom |
| Scroll Mode Toggle | LJ_Up+RJ_Left |
| Show Attributes Toggle | LJ_ Bottom +RJ_Center |
| Grade 2 Braille Toggle | LJ_Center+RJ_Center |
| Braille Tracking Toggle | LJ_ Bottom +RJ_ Bottom |
| Spacing Toggle | LJ_ Bottom +RJ_Right |
| Attributes To Show Rotor | LJ_Up+RJ_Center |
| Display Individual Attribute Rotor | LJ_Center+RJ_Up |
| Quick Message Rotor | LJ_ Bottom +RJ_Left |

# 5. Using the Seika Studio with NVDA

## 5.1 Pairing Braille Devices

There is no need to install the add-on for NVDA 2021.2 and later version.

You should install the add-on if the versions lower than NVDA 2020.2.

Please select the “Seika Notetaker” item in the Braille settings of the NVDA to use the Seika Studio.

Set the Seika Studio to USB connect or Bluetooth connect mode by using Left Button + Routing 2 or Left Button + Routing 3 respectively. Please refer to Section 2.2 for more connection steps.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

## 5.2 Braille Commands

| Function | Keys |
| --- | --- |
| Braille_scroll Back | LB |
| Braille_scroll Forward | RB |
| Braille_previous Line | LJ_Up |
| Braille_next Line | LJ_ Bottom |
| Braille_toggle Te ther | LJ_Center |
| Show gui | RB+LB |
| Tab | LJ_Right |
| Shift+tab | LJ_Left |
| Up arrow | RJ_Up |
| Down arrow | RJ_ Bottom |
| Left arrow | RJ_Left |
| Right arrow | RJ_Right |
| Enter | RJ_Center |

# 6. Using the Seika Studio Bluetooth with VoiceOver

## 6.1 Using VoiceOver on the iPad and iPhone

Turn on the Seika Studio by power switch and select the Bluetooth connection mode by using Left Button + Routing 3. Once Seika Studio Bluetooth is started, it will display “wait bt connect...”, two seconds later, it will display “tsm **** pin 0000”, where **** is the Bluetooth name serial number.

Make sure your iPhone Bluetooth is turned off.

Enable the voiceover by the following steps: iPhone Settings -> General -> Accessibility -> VoiceOver.

Enable the Braille by the following steps: VoiceOver -> Braille. When you are asked whether open Bluetooth or not, please select open Bluetooth, then iPhone will search the Seika Studio, and it will take about one minutes. When iPhone find Seika Studio and display Bluetooth name TSM ****, you need double click this item and input the pin code 0000 to set up the connection.

If the iPhone or iPad is sleep or out of range, or you restart Seika Studio, the connection maybe broken. You can double click the TSM xxxx in VoiceOver ->Braille->Choose a braille display and set up the connection again.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

If you would like to switch two iOS devices connection with Seika Studio, you can make one iOS device sleep and the Seika Studio will wait connection. Then you can start the other iOS device and double click the TSM xxxx in VoiceOver ->Braille->Choose a braille display, the connection will be set up with this iOS device.

By default, VoiceOver displays uncontracted, eight-dot braille. In this mode, VoiceOver automatically translates each character and digital you type on your braille display into braille output, as you type it. You can turn off eight-dot braille to use uncontracted, six-dot braille instead. In this mode, VoiceOver automatically translates each word you type on your braille display into braille output, after it determines you’ve completed a word or you press the Space bar on your braille display.

## 6.2 Using VoiceOver on a Mac OS X computer

Turn on the Seika Studio by power switch and select the Bluetooth connection mode by using Left Button + Routing 3.

Launch VoiceOver Utility. Click on the "Braille" tab on the left. Click on the "Displays" tab. The Braille display Bluetooth name will appear in the list of Braille devices. Its name will begin with TSM in the list of devices. Click the device name to select it, and initiate the pairing. You will be taken to a passcode entry screen, enter the passcode 0000.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

## 6.3 Navigation keys and commands

| Function | keys |
| --- | --- |
| Move to previous item | 1 + Space |
| Move to next item | 4 + Space |
| Move to previous item using rotor setting | 3 + Space |
| Move to next item using rotor setting | 6 + Space |
| Select previous rotor setting | 2-3 + Space |
| Select next rotor setting | 5-6 + Space |
| Move to the first element | 1-2-3 + Space |
| Move to the last element | 4-5-6+ Space |
| Read page starting at selected item | 1-2-3-5 + Space |
| Read page starting at the top | 2-4-5-6+ Space |
| Activates the Home button | 1-2-5 + Space |
| Goes to the Status bar | 2-3-4+ Space |
| Activates the Volume Up button | 3-4-5 + Space |
| Activates the Volume Down button | 1-2-6 + Space |
| Activates a Back button if present | 1-2 + Space |
| Activates the Eject key | 1-4-6 + Space |
| Activates the Delete key | 7 + Space |
| Activates the Delete key | 1-4-5 + Space |
| Activates the Return (Enter) key | 8 + Space |
| Activates the Return (Enter) key | 1-5 + Space |
| Activates the Tab key | 2-3-4-5+ Space |
| Toggle Screen Curtain on and off | 1-2-3-4-5-6 + Space |
| Pause or Continue speech | 1-2-3-4+ Space |
| Toggle speech on and off | 1-3-4 + Space |
| Speak page number or rows being displayed | 3-4+Space |
| Scroll up one page | 3-4-5-6+Space |
| Scroll down one page | 1-4-5-6+Space |
| Scroll left one page | 2-4-6+ Space |
| Scroll right one page | 1-3-5 + Space |
| Deselect text | 2-3-5 + Space |
| Select text | 2-5-6 + Space |
| Select all | 2-3-5-6+Space |
| Cut | 1-3-4-6+Space |
| Copy | 1-4+Space |
| Paste | 1-2-3-6+Space |
| Undo typing | 1-3-5-6+Space |
| Redo typing | 2-3-4-6+Space |
| Pan Braille to the left | 2 + Space |
| Pan Braille to the right | 5 + Space |
| Goes to Notification Center | 4-6+ Space |
| Goes to Control Center | 2-5+Space |
| Toggle announcement history | 1-3-4-5 + Space |
| Switch between contracted and uncontracted Braille | 1-2-4-5 + Space |
| Move to previous container (iPad only) | 1-7+Space |
| Move to next container (iPad only) | 4-7+Space |

Device keys for VoiceOver navigation:

| Function | keys |
| --- | --- |
| Pan Braille to the left | LB |
| Pan Braille to the right | RB |
| Select item under your finger | LJ_Center |
| Move to previous item | LJ_Left |
| Move to next item | LJ_Right |
| Move to previous item using rotor setting | LJ_Up |
| Move to next item using rotor setting | LJ_ Bottom |
| Select item under your finger | RJ_Center |
| Move to previous item | RJ_Left |
| Move to next item | RJ_Right |
| Move to previous item using rotor setting | RJ_Up |
| Move to next item using rotor setting | RJ_ Bottom |
| Double tap the selected item | Router |
| Toggle expanded status description | Status Router |

# 7. Using the Seika Studio with android Talkback

Your phone must be Android 4.1 or above it, and please download and install Talkback and Braille Back.

Firstly, turn on the Seika Studio by power switch and select the Bluetooth connection mode by using Left Button + Routing 3.

Secondly, turn on your phone Bluetooth and search the Bluetooth device of TSM **** (where **** means Seika Studio Bluetooth serial number). If need, please enter the pin code “0000”.

Change the keyboard to QWERTY or 8-key Braille keyboard by using the slide switch. The far away from your side position of slide switch will lock the keyboard for accidently inputting, middle position will active the QWERTY keyboard, near your side position will active the 8-key Braille keyboard which uses asdfjkl; and space for emulating. There will beep tone Mi after select lock keyboard, beeps tone Mi-Mi after select QWERTY keyboards, and beeps tone Mi-Mi-Mi after select 8-key Braille keyboard.

You should pair the QWERTY Bluetooth keyboard of the Seika Studio if use the QWERTY keyboard in Bluetooth connection mode. There is no need to pair the 8-key Braille Bluetooth keyboard. Open the Bluetooth in your device, search and connect the QWERTY Bluetooth keyboard of Seika Studio named “KMBT-XY”, XY is the keyboard identity number. You can only search and connect the QWERTY Bluetooth keyboard of Seika Studio in the Bluetooth connection mode. There will be auto connect after the first settings of the QWERTY Bluetooth keyboard.

| Function | Keys |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter, RJ_Center, LJ_Center, Cursor-Routing in Braille Cursor mode |
| Activate the main screen | Backspace + dots125(h) |
| Back(Escape) | Space + dots12(b) |
| Insert Space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Braille Backspace | Backspace |
| Clear/Delete | Dots7 |
| Braille Pan Left | LB |
| Braille Pan Right | RB |
| Previous Character/Grid Item/Tab Sheet | LJ_Left,RJ_Left,Space + dot3 |
| Next Character/Grid Item/Tab Sheet | LJ_Right,RJ_Right,Space + dot6 |
| Previous Line/List Item/Menu Item | LJ_Up,RJ_Up,Space + dot1 |
| Next Line/List Item/Menu Item | LJ_Bottom, RJ_ Bottom,Space + dot4 |
| Previous Word | Space+LJ_Left,Space+RJ_Left, Space+dot2 |
| Next Word | Space +LJ_Right, Space + RJ_Right,Space + dot5 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Space + Backspace + dot3 |
| Hang-Up/End Call | Space + Backspace + dot6 |
| Application Menu (Switch Application) | LJ_Center+RJ_Center, Space+ dots134(m) |
| List of Open Applications(Show Start Menu) | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots24(i) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile (Switch Profile) | Space + dots1234(p) |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + Backspace + dots1236(v) |
| Increase Volume | Space + Enter + dots1236(v) |
| Decrease Speed | Space +Backspace + dots234(s) |
| Increase Speed | Space+ Enter + dots234(s) |
| Change Verbosity Level | Space + dots1236(v) |
| Change Keyboard Echo | Space + Enter + dots15(e) |
| Read Date and Time | Space + dots2345(t) |
| Toggle Speech Mute | Space + Backspace + dots136(u) |
| Interrupt/Stop Speech | Space + Backspace + dots24(i) |
| Toggle Command Help Mode | Space + dots125(h) |
| Braille Routing | Routing |

# 8. How to re-install the Windows 64-bit OS on the Seika Studio

Caution: you should backup all the data of the Seika Studio before re-install the Windows OS.

Caution: there is no Screen Reader can be run when re-install the Windows OS.

## 8.1 How to make a bootable Windows 64-bit OS USB disk

1. Prepare an empty USB disk at least 8 GB for install the Windows OS. Connect the USB disk to your Windows OS PC.

2. On your PC, download the Windows media creation tool from Microsoft website:

https://www.microsoft.com/en-us/software-download/windows10

Or:

https://www.microsoft.com/en-us/software-download/windows11

3. On your PC, double click on the Windows media creation tool and start the creation.

4. Select “Create installation media for another PC”.

5. Make sure select “64-bit(x64)” from the architecture item.

6. Select USB disk and press “Next” and then select the USB disk which already plug in your PC.

7. Follow the prompt and wait. Then it will create a bootable USB disk with Windows 64-bit OS.

## 8.2 How to re-install a Windows 64-bit OS on your Seika Studio

1. Make sure you have back up all your data of the Seika Studio.

2. Turn off the power switch and connect the wall adapter with the Seika Studio.

3. Connect the bootable USB disk on your Seika Studio USB slot.

4. Connect the USB mouse on your Seika Studio USB slot.

5. Connect the Seika Studio HDMI with your monitor screen.

6. Select the QWERTY keyboard by setting to the middle position of the slide switch.

7. Turn on the power switch and press the Windows computer button to turn on the internal computer, and quickly cycle press and release the F10 button on the keyboard of the Seika Studio in order to jump to the bootable USB disk.

8. Use the arrow key on the keyboard of the Seika Studio and select the bootable USB disk.

9. Press Enter on the bootable USB disk and Seika Studio will restart to install the windows OS.

10. If you have no Windows product key yet, you should select “I don’t have a product key” and the installation will continue.

11. Option step: delete the entire partitions of the “Drive 0” when the “Where do you want to install the Windows?” prompt. Make sure you have back up all your data of the Seika Studio. After delete all the partitions on the “Drive 0”, all data will be delete and you can install a clean Windows OS.

12. Select the “Drive 0” for the Windows 64-bit OS installation location and press “Next”.

13. After install the Windows OS, you should install all the drivers, please download here:

https://www.intel.com/content/www/us/en/support/detect.html