Mini Seika

Braille Display

User Manual

Version 1.33

2014 Aug.

Preface

Thank you for purchasing the Mini Seika Braille Display.

The Mini Seika is a portable 16 cell Braille display with an integrated Braille keyboard.

Before using the Mini Seika, please read this user manual.

If you have additional questions about using the Mini Seika, please contact distributors by e-mail, letter, fax, or telephone.

Safety Precautions

For your own safety and to protect the Mini Seika from accidental damage, please read and abide by the following safety precautions.

1. The input voltage of the AC adapter is 100V - 240V, and the output is DC 5V / 1A.

2. Your Mini Seika needs to be handled with care. Make sure to handle the unit in a proper manner. Also, the Mini Seika is very sensitive to dust. Make sure to keep Mini Seika away from dusty environments.

3. Please keep your Mini Seika away from a long time in a closed, high temperatur environments, such as inside a car on a hot summer day

4. Do not disassemble the Mini Seika yourself.

Table of Contents

1. Getting Started	6

1.1 Equipment Checklist	6

1.2 Device description	7

1.3 Power Switch and Battery	10

1.4 Hardware Specifications	11

2. Basic Functions of the Mini Seika	12

2.0.1 How to Use the Commands	12

2.0.2 General Functions	12

2.1 Notepad	14

2.2 File Manage	24

2.3 Read	26

2.4 USB connect mode	31

2.5 Bluetooth mode	32

2.6 Clock	32

2.7 Calculator mode	34

2.8 Tools mode	36

*2.9 Transfer file between Mini Seika and computer	39

3. Configuring the Bluetooth Connection	40

3.1 Installing the Bluetooth Dongle Driver	41

3.2 Setting up the Bluetooth Connection	41

4. Using the Mini  Seika with JAWS	43

4.1 Installing the Seika Patch for JAWS	43

4.2 Working in JAWS	43

4.3 Uninstalling the patch for JAWS	47

5. Using the Mini  Seika with Window-Eyes	48

5.1 Configuring Window-Eyes 7.5.2	48

5.2 Navigation Keys and Commands	49

6. Using the Mini  Seika with VoiceOver	50

6.1 Using VoiceOver on the iPad and iPhone	50

6.2 Using VoiceOver on a Mac OS X computer	51

6.3 Navigation keys and commands	51

7. Using the Mini  Seika with NVDA	55

7.1 Pairing Braille Devices	55

7.2 Braille Commands	55

8. Using the Mini Seika with Mobile Accessibility v2.0 for Android	57

8.1 Pairing Braille Devices	57

8.2 Configuring Braille Display with Mobile Accessibility	57

8.3 Removing Braille Display configuration	58

8.4 Enabling / Disabling Braille	58

8.5 Braille Commands	59

9. Using the Mini  Seika with Mobile Speak for Symbian	63

10. Using the Mini Seika with TALKS	66

11. Using the Mini Seika with android Talkback	69

# 1. Getting Started

Welcome to the Mini Seika, an innovative personal productivity system. The Mini Seika is a portable 16 cell Braille display with a notetaking feature as well as a suite of additional powerful applications. The Mini Seika is compatible with several popular screen reading software programs that are used with mobile phones, PC’s, and Apple devices via Bluetooth or USB. The Mini Seika also includes the following applications: Notepad, File Management, Book Reader, Clock and Calculator.

## 1.1 Equipment Checklist

Carefully unpack your Mini Seika. Save the box and packing materials for future use. Your Mini Seika package contains the following items:

- Mini Seika in its leather carrying case

- carrying case with a strap

- Power adapter (the output is 5V/1A)

- USB Cable

- MicroSD card which comes preinstalled in the Mini Seika

- USB disk

- Documentation/driver CD

- User manual

## 1.2 Device description

The Mini Seika total weight is 0.7 lbs with dimensions of 6.3 x 3.78 x 0.9 inches.

The Mini Seika can operate from its internal rechargeable battery or from its AC adapter via USB cable.

The Mini Seika comes in a leather carrying case. To open the case, lift the flap under the front edge and fold it back.

Place the Mini Seika on a flat surface with the Braille display facing you.

On the left side, you’ll find from back to front:

- Avertical USB slot which is for the USB disk

- A horizontal slot for the MicroSD card. The MicroSD card is already inside the Mini. Plugging/unplugging the MicroSD card should be done carefully. The MicroSD card slot has an auto-locking feature - just press, and then release it in order to unlock the card.

- Do not remove the MicroSD card while the device is working. Please turn off the Mini Seika before removing the MicroSD card.

On the right side, from back to front you’ll find:

- A mini USB interface

- A rocker switch to power on/off the Mini Seika. To turn the Mini Seika on, press the rocker switch toward you.

On the top surface of the Mini Seika you’ll find:

- 16 refreshable braille cells ((8 dots))

- Corresponding to the braille cells, there are raised camber ridges through the rear of each cell. These buttons are called the cursor routing keys. They are especially useful when editing a document. In Notepad, for example, when you press a touch cursor button, the cursor moves to that cell. You can then edit the character displayed on that cell. The touch cursor can also be used to select an item from a menu or list.

- Behind the braille cells are the 8 Braille keys, named dot key 1 through 8. In the front of the 16 cells, there are two keys, named Space (key 9, key A). When you are typing, you should place your fingers on these 9 keys in the following manner: Place the index finger, middle finger, ring finger, and pinky finger of your left hand on the left four buttons. Then, place the index finger, middle finger, ring finger, and pinky finger of your right hand on the left four buttons. Finally, place both of your thumbs on the spacebar. The button where you placed your left index finger corresponds with dot 1 in Braille, The button that your left middle finger is on corresponds to dot 2, and button under your left ring finger corresponds to dot 3. the button on which your left pinky finger is placed is the backspace key (key 7). The button under your right index finger corresponds with dot 4 in Braille, the button under your right middle finger is dot 5, and the button on which your ring finger is placed corresponds with dot 6 in Braille. The button on which your right pinky is placed is the Enter key (key 8). And, the two buttons under your thumbs are the spacebar.

- On each side of the braille display are two streamline shaped keys, these are the Left Button (LB) and Right Button (RB).

- On the same row as the space are two five-direction Navigation joysticks (left, right, up, down and center). In this manual, they will be referred to as “LJ” for the left joystick and “RJ” for the right joystick.

## 1.3 Power Switch and Battery

As you move your hand along the right side of the case towards the rear, you will find a rocker switch surrounded by a concave rectangular frame. To switch the Mini Seika on, press the rocker switch toward you. The Mini Seika immediately beeps twice and “notepad” appears on the braille display. This is the first item in the main menu. To turn off the Mini, press the rocker switch away from you.

If you don’t use the Mini Seika for approximately 1 minute, it will enter sleep mode. Pressing any key will end the sleep mode, but it is recommended that you momentarily press the Joystick left or Joystick right. This will avoid accidentally inserting characters in an open document. You can set the waiting time from 1 minute to 30 minute, please refer to Chapter 2.8.5 Set sleep.

If you don’t use the Mini Seika for approximately 4 hours, it will save the active file automatically (the unnamed file will be saved as “temp” file name in the current folder) and turn off to save power, please note that you CANNOT wake it up by pressing keys except restart Mini Seika.

The Mini Seika can operate from its internal rechargeable battery or from its AC adapter. The length of time you can run the Mini Seika from a fully charged battery is typically over 10 hours before it becomes flat. Activities like using the USB disk or Bluetooth will increase the drain on the battery.

When the battery level falls below 15%, the Mini displays the message, “please charge” and beeps tone “Mi-Lai-Do” suggests you connect the AC adapter. The power socket is the mini USB socket, which is located on the right side, behind the power switch. Plug the smaller end of the USB cable into the Mini Seika, and plug the larger end of the USB cable into the USB port on the AC adapter. When you connect the AC adapter successfully, it will display “usb charging” and you will hear beeps tone “Do-Lai-Mi” which indicates the battery is charging.

If the battery is flat and the AC adapter is connected, the battery will charge in about 4 hours. Charging takes less time if the battery is only partially discharged. You can charge the battery and use the Mini Seika simultaneously although the battery may take longer to charge. Please ensure that the battery charges correctly, you can only charge in ambient temperatures between 41 and 95 degrees Fahrenheit, or 5 to 35 degrees Celsius.

## 1.4 Hardware Specifications

- Memory extension: MicroSD Card and U-disk.

- CPU: ARM

- Battery: Lithium rechargeable battery

- Keyboard: 8-Dot Braille keyboard, 2 navigation keys, 16 cursor routing keys, 2 navigation joysticks,2 spaces

- Braille display: 16 refreshable 8-Dot Braille cells

- Wireless: Bluetooth

- Interface: USB, Mini USB, MicroSD card slot

# 2. Basic Functions of the Mini Seika

### 2.0.1 How to Use the Commands

All of the commands used on the Mini Seika are comprised of different combinations of the Braille keys. In this user manual, the “+” sign is used to indicate key combinations. For example, “Space +z” indicates that the user should press the Space key and letter z braille (dots 1-3-5-6) simultaneously and then release them at the same time. You can execute most commands by pressing the “Enter” button (press Joystick_Center, or press Dot8). If you would like to cancel the execution of any command, or return to the previous status, press “Space +e” (1-5 + Space).

Note: the professional version function is marked by “*” (Star), please contact the distributor to get professional firmware.

### 2.0.2 General Functions

Upon switching on the unit, it will beep twice, and the first item in the main menu appears on the braille display: Notepad.

The main menu includes: Notepad, File Management, Read, USB connect, Bluetooth, Clock, Calculator, Tools. Use the Joystick_Up and Joystick_Down to scroll items in the main menu and press Enter or Joystick_Right or cursor routing keys above the item to select it. You can type the first letter to jump to the corresponding item, for example, press the letter “r” (dots 1-2-3-5) to jump to the “read” mode.

Note:As the clock and calculator menu have the same first letter, you can type the letter “a” to jump directly to the calculator.

Several menu items have sub menus, for example: “tools”, will display in braille as (tls) with the sub menu displayed after the punctuation sign >, as follows: tls>power.

tls>power.

Open the sub menu power by pressing Enter or Joystick_Rightor the cursor routing keys above the item. For example: press the cursor routing keys above (power). Escape by using Joystick_Left or press Space +e (1-5 + Space) or press the cursor routing keys above the letters (tls) which will return you to the main menu.

Tree views are used to display the structure of folders and files on the MicroSD card. The file name begins with the letter f, and the folder name begins with the letter d. If you press the Enter or Joystick_Right on the folder, it will display the list of files within the tree view of the folder structure. If you press Enter on the file, it will open the file.If you want to move up one level in the tree view, press the BackSpace (7) or Space +e (1-5 + Space) or Joystick_Left.

The Mini Seika is available in several languages and coordinating braille tables. Switch between different languages and braille tables by pressing Space + l (1-2-3 + Space) in the main menu. Use Joystick_Up and Joystick_Down to scroll between different languages, such as “language: usa”, “language: en”,  “lingua:italiano”(Italian), “langue:francaise”(French), “اللغة:عربي”(Arabic), "sprache:Deutsch"(German), "jezik:hrvatski"(Croatian), "språk:norsk"(Norwegian), “language: Japan”(Japanese), “語言:中文粵語”(Cantonese), “ngônngữ: việt”(Vietnamese), “language: taiwan” and press Enter to select it. You can also press Space +e (1-5 + Space) to cancel switching languages and braille tables.

## 2.1 Notepad

The Notepad application allows you to create and edit simple files; each file size can’t exceed 900k bytes. Notepad supports long file names and multi-level folders. If the file name is too long to display at one time, you can use Left Button and Right Button to display the whole file name, file size, modification date and time. All file names are displayed and edited using 6-Dotbraille.

In order to use the Notepad, you will need to have a MicroSD card in the card slot (which is already installed when you receive the Mini)or a USB disk in the USB slot. The Notepad has two sub menus, MicroSD card and U-Disk. When you start the Notepad application, you need select the SD card or U-Disk, and then the Mini Seika will display the first file name on the MicroSD card or U-Disk, such as “help_en.txt”. Use the Joystick_Up and Joystick_Down to scroll the file name and press Enter or cursor routing keys above the item to select it. You can also press BackSpace or Joystick_Left or press Space +e (1-5 + Space)to move back .If there isn’t a file on the MicroSD card or U-Disk, the Mini Seika will display “no content”. If there is not a  MicroSD card or U disk, it will display “no SD card” or “no U disk” message. Press Enter or Space +e (1-5 + Space) to go back to the menu.

All braille files (brl, brf and so on) are displayed and edited using 6-Dot braille. The txt file is displayed and edited using 8-Dot Braille. Press Space +p (1-2-3-4 + Space) to switch between 6-Dot and 8-Dot braille.

NOTE: For faster dealing with file, you can execute filemanage command in the Notepad file tree list (such as file rename, delete, cut, copy, paste), please refer to Chapter 2.2 for more commond details

2.1.1 New File (Space +n; 1-3-4-5 + Space)

This command will close any open document and create a new document which you can immediately edit. The document which you do not save will be prompted to save.

Press Space + n (1-3-4-5 + Space) to start a new document which you can immediately edit, after you input some word, you can press Space + s (2-3-4 + Space) to save the file in the current folder and the Mini Seika will display “input f name.ext” prompt. After input the file name and extension name, and pressing Enter, you will return to the current open file. If you only input the file name and pressing Enter, the Mini Seika will add default extension file name automatically. You can also press Space +e (1-5 + Space) to cancel input file name and go back to continue to edit the current file. You cannot use the following characters in a file name: “|, \, <, >, ?, :, *”

2.1.2 Close File (Space +e; 1-5 +Space)

This command will close any open document.

You can press Space + e (1-5 +Space) to exit the editing document, you will be prompted to save that document. The Mini will display “save file? y n c”, which y means save, n means not save, c means cancel and return you to the currently open file. You can press Joystick_Left or Joystick_Right to move the cursor under y, n or c and press the Enter key to confirm. Or you can also input the letter y, n or c. Or you can press the press the routing cursor button above the y, n, or c on the braille display.

When you exit a file, it will automatically set a mark. When you re-open this file next time, it will automatically start at the mark location.

2.1.3 Save File (Space +s; 2-3-4 + Space)

This function allows you to save the document under its current name. Press Space + s (2-3-4 + Space), the Mini Seika will display “save ok” prompt. You can press any key to return to the current file. If you have not yet named the document, you will be prompted “input f name.ext”, after inputting the file name and extension name and pressing Enter, you will return to the currently open document. If you only input the file name and pressing Enter, the Mini Seika will add default extension file name automatically. You can also press Space +e (1-5 + Space) to cancel creating a file name and to continue editing the current file. If you want to save a new file under a different folder, choose the folder before saving the file. When you open Notepad, press Space + Enter + f (1-2-4 + Space + Enter) which will bring you to the list of files and folders in the tree view. Select the folder you want to place the file into and press Enter. Press Space + n (1-3-4-5 + Space) to create a new folder. You can press Space +e (1-5 + Space) to confirm the current name and go back to continue to edit the file. Press BackSpace to move to an Upper-level within the tree view.

2.1.4 Save As (Space + Enter + s; 2-3-4 + Space + Enter)

This command allows you to save the document under a different folder. Press Space + Enter + s (2-3-4 + Space + Enter) to go to folder tree, after you select the right folder position, press Space + Enter + s (2-3-4 + Space + Enter) again to save, and you will return to the current file. You can press Space + n (1-3-4-5 + Space) to create a new folder. Press Space +e (1-5 + Space) to cancel “save as” command.

2.1.5 Fast Quit (Space +q; 1-2-3-4-5 + Space)

This command closes any open document and returns to the main menu. The document which you do not save will be prompted to save.

When you exit a file, it will automatically set a mark. When you re-open this file next time, you are returned to your previous position in the document.

2.1.6 Find (Space +f; 1-2-4 + Space)

This command allows you to search backwards or forwards for a letter-sequence, starting from the cursor location. Once the text is found you are taken to that position in your document. To execute this command, press Space+ f (1-2-4 + Space), the Mini will display “input find text”, input the letter-sequence (for example: mini) and press Enter to start a search. If there are no search results, “not found” is displayed and you are returned to your previous position in the document.

2.1.7 Find Next (Space+Enter+f; 1-2-4+Space+Enter)

This triggers a search (with the same word you input in Find command) moving forward to the end of the document.

2.1.8 Find Previous (Space +Backspace+f; 1-2-4 + Space+Backspace)

This triggers a search (with the same word you input in the Find command) moving back to the beginning of the document.

2.1.9 Set Mark (Space + m; 1-3-4 + Space).

This command marks the position in the document that you want to return to later. This allows you to return to the same position within a document even after it has been closed and re-opened. To execute this command, move the cursor to the location at which you want to set a mark, and press Space+m. The mark position is kept in a hidden file on the MicroSD card or U-Disk.

You can set up to 10 marks in a document on the Mini Seika

2.1.10 Jump to Mark (Space + j; 2-4-5 + Space)

This command will take you to the mark position.

If you want to go to next mark, press Space+j or Space+enter+j. If you want to go to previous mark, press Space+Backspace+j (2-4-5-7+Space).

2.1.11 Delete Mark (Space+Enter+d; 1-4-5-8 + Space)

When you jump to a Mark, press Space+Enter+d (1-4-5-8 + Space), the Mark will be delete.

2.1.12 Selecting Text (Space+Enter; 8 + Space)

The Selection command allows you to select a certain portion of text that you would like to edit. When you use the Selection command, the dots 7-8 will be up to indicate what text is being selected.

To select text in a document, place your cursor on the location at which you want to start selecting text, Press Space+Enter, then all dots in the current letter will flicker . The end of your selection is the position at which you place your cursor after Pressing Space+Enter.

2.1.13 Copy and Paste (Space+c; 1-4 + Space)

To copy the selected text to the clipboard, press Space+c (1-4+Space).

2.1.14 Paste (Space+v; 1-2-3-6+Space)

To paste the text from the clip board to your current cursor location, press Space+v (1-2-3-6+Space).

2.1.15 Cut (Space+x; 1-3-4-6+Space)

While editing a document, pressing Space+x (1-3-4-6+Space) activates the Cut command.

The difference between Cut and Copy is that if you execute the Cut command, the selected text is moved to the clipboard and the selection is removed from the document. The Copy command simply copies the selection to the clipboard, and the selected text remains in the document.

2.1.16 Delete (Backspace, Space+d; 7, 1-4-5+Space)

Delete previous character, press “Backspace” (7).

Delete character, press space+d (1-4-5+Space)

Delete block: space+d (1-4-5+Space). The Delete block command is only available when there is a block of text selected. You can also access this command via the “Edit” Menu.

2.1.17 Get current position information (Space + =; 1-2-3-4-5-6+Space)

You can get the current position information by pressing space+= (1-2-3-4-5-6 + space).

Braille and Hot Key Summary

| Action | Hot key | Braille |
| --- | --- | --- |
| Space |  | 9, A, 9A |
| BackSpace |  | 7 |
| Enter | Joystick_Center | 8 |
| Escape | Space + e | 1-5 + Space |
| Fast exit to main menu | Space + q | 1-2-3-4-5 + Space |
| New file | Space + n | 1-3-4-5 + Space |
| Save file | Space + s | 2-3-4 + Space |
| Save as file | Space + Enter + s | 2-3-4-8 + Space |
| Toggle 6/8 dots | Space + p | 1-2-3-4 + Space |
| Display current time | Space + t | 2-3-4-5 + Space |
| Setting current folder | Space + Enter + f | 1-2-4+ Enter + Space |

Edit Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Dot input key |  | 1 through 8 |
| Space |  | 9, A, 9A |
| BackSpace |  | 7 |
| Enter | Joystick_Center | 8 |
| Close and exit file | Space + e | 1-5 + Space |
| Delete one character | Space + d | 1-4-5 + Space |
| Find | Space + f | 1-2-4 + Space |
| Find Next word | Space + x, LJ_Up + RJ_Right | 1-3-4-6 + Space |
| Find Previous word | Space + y, LJ_Up + RJ_Left | 1-3-4-5-6 + Space |
| Set Mark | Space + m | 1-3-4 + Space |
| Delete Mark | Space+Enter+d | 1-4-5-8 + Space |
| Jump to Mark | Space + j | 2-4-5 + Space |
| Delete Mark | Space+Enter+d | 1-4-5-8 +Space |
| Jump to Next mark | Space+Enter+j | 2-4-5-8 + Space |
| Jump to Previous mark | Space+Backspace+j | 2-4-5-7+Space |
| Set the block starting point | Space+Enter | 8+Space |
| Set the end | Space+Enter | 8+Space |
| Cut | Space+x | 1-3-4-6+Space |
| Delete | Space+d | 1-4-5 + Space |
| Copy | Space+c | 1-4+Space |
| Paste | Space+v | 1-2-3-6+Space |
| Get current position | Space+= | 1-2-3-4-5-6 + Space |

Navigation Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Pan Left | LB |  |
| Pan Right | RB |  |
| Pan left 5 screen | Space + LJ_Left | 1-3 + Space |
| Pan right 5 screen | Space + LJ_Right | 4-6 + Space |
| Move to next word | LJ_Right | 5 + Space |
| Move to previous word | LJ_Left | 2 + Space |
| Cursor goes to beginning of previous sentence | RJ_Up | 1 + Space |
| Cursor goes to beginning of next sentence | RJ_Down | 4 + Space |
| Cursor goes to beginning of current sentence |  | 1-4 + Space |
| Cursor goes to beginning of previous paragraph | LJ_Up | 2-3 + Space |
| Cursor goes to beginning of next paragraph | LJ_Down | 5-6 + Space |
| Cursor goes to beginning of current paragraph |  | 2-3-5-6 + Space |
| Cursor goes to beginning of document | Space + LJ_Up | 1-2-3 + Space |
| Cursor goes to end of document | Space + LJ_Down | 4-5-6 + Space |
| Cursor goes left | RJ_Left | 3 + Space |
| Cursor goes right | RJ_Right | 6 + Space |
| Cursor goes to corresponding cell | Routing button1 through 16 |  |

- 2.2 File Manage

This function enables you to manage the files and folders in the Micro SD card or U-Disk. The filemange has two sub menus, SD Card and U-Disk. Select one of these in order to manage the files on the Micro SD card or USB disk.

Filemange>SD

Filemange>USB

This function enables you to browse in the file tree list, you can use Joystick_Up and Joystick Down to scroll through different files, press LB or RB to display the file size, modified time and date. Press Enter or Joystick_Right in the File Manage menu to open the folder or the file..

2.2.1 New Folder (Space+n; 1-3-4-5+Space)

This function enables you to browse in the file list and create a new folder.

Press Space + n (1-3-4-5 + Space) to create a new folder in the tree list, the Mini Seika will display “input d name” prompt. You can input the new folder name, and then press the Enter to create the new folder, or Space +e (1-5 + Space) to cancel.

2.2.2 Delete (Space + d; 1-4-5 + Space)

When you press command Space + d (1-4-5 + Space) in a file or folder, the Mini Seika will display “delete?”, press Enter to delete file or folder, press Space +e (1-5 + Space) to cancel the deletion. If you want to delete a folder, please delete all files in this folder firstly.

2.2.3 Rename (Space +r; 1-2-3-5 + Space)

Press Space +r (1-2-3-5 + Space), the Mini Seika will prompt you to input the new name for the file or folder. After editing the name, press Enter to save the new file or folder name. If there is the same file or folder name in the current folder, the Mini Seika will display “exists, replace?”, press Enter to replace, press Space +e (1-5 + Space) to cancel renaming the file.

2.2.4 Copy (Space+c; 1-4+Space), Cut (Space+x; 1-3-4-6+Space) and Paste (Space+v; 1-2-3-6+Space)

These commands allow you copy, cut and paste file between different folders, and also between Micro SD card and U-disk.

To copy the selected file to the clipboard, press Space+c (1-4+Space).

To cut the file to the clipboard, press Space+x (1-3-4-6+Space).

To paste the file from the clipboard to your current folder location, press Space+v (1-2-3-6+Space).

File Manage Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Create a new folder | Space+ n | 1-3-4-5 + Space |
| Delete file or folder | Space+d | 1-4-5 + Space |
| Rename file or folder | Space+r | 1-2-3-5+ Space |
| Copy file | Space+c | 1-4+Space |
| Cut file | Space+x | 1-3-4-6+Space |
| Paste file | Space+v | 1-2-3-6+Space |

- 2.3 Read

The Read function enables you to read the files in the U-Disk or Micro SD card, and the file’s size should not exceed 900k bytes. In order to use the Read function, you need to place a Micro SD card in the card slot or a USB disk in the USB slot. The Mini  Seikacan read the brl, brf, bse, txt files and so on. If the file extension name is txt, the default display setting is 8-Dotbraille. If the file extension name is brl, brf, bse or others, the default display setting is 6-Dotbraille.The Mini  Seikasupports long file names and multi-level folders. If the file name is too long to display at one time, you can use LB and RB to display the whole file name, file size, modification date and time.When you exit the file, the mark will be added in the current place automatically.

The Read item has two sub menus, SD Card and U-Disk. Select one of these in order to read the files on the Micro SD card or USB disk.

2.3.1 Auto scroll (Space + Right Joystick_Right)

The auto scroll command scrolls automatically. You can turn the auto scroll feature on by pressing Space + Right Joystick_Right, and stop the auto scroll by pressing Enter. For faster scrolling, press the Right Joystick_Right. For slower scrolling, press the Right Joystick Left .

2.3.2 Close File (Space +e; 1-5 +Space)

This command will close any open document.

When you exit a file, it will automatically set a mark. When you open this file next time, it will automatically start at the mark location.

2.3.3 Fast Quit (Space +q; 1-2-3-4-5 + Space)

This command closes any open document and returns to the main menu.

When you exit a file, it will automatically set a mark. When you open this file next time, it will automatically start at the mark location.

2.3.4 Find (Space +f; 1-2-4 + Space)

The find function allows you to search for text in a document.  Once the text is found you will be taken to that position in your document. Press Space+ f (1-2-4 + Space), the Mini  Seika will display “input find text” prompt, input in the letter-sequence (for example: apple) and press Enter to start a search. If there are no search results, “not found” is displayed and you are returned to your previous position in the document.

2.3.5 Find Next (Space+Enter+f; 1-2-4+Space+Enter)

Start a search (with the same word you input in the Find command) moving forward to the end of the document.

2.3.6 Find Previous(Space +Backspace+f; 1-2-4 + Space+Backspace)

Start a search (with the same word you input in the Find command) moving to the beginning of the document.

2.3.7 Set Mark (Space + m; 1-3-4 + Space).

This command marks the current Location in the document. This function allows you to return to the same position within a document even after it has been closed and re-opened. The mark position is kept in a hidden file on the Micro SD card or U-Disk.

*You can set up to 10 marks in a document on the Mini  Seika.

2.3.8 Jump to Mark (Space + j; 2-4-5 + Space)

This command will take you to the mark position.

*If you want to go to next mark, press Space+j. If you want to go to previous mark, press Space+Backspace+j (2-4-5-7+Space+Backspace).

Note: Do not pull out the Micro SD card when you are editing files. If you need to pull out the Micro SDcard, please turn the Mini  Seika off.

Read Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Pan Left | LB |  |
| Pan Right | RB |  |
| Pan left 5 screen | Space + LJ_Left | 1-3 + Space |
| Pan Right 5 screen | Space + LJ_Right | 4-6+ Space |
| Toggle 6/8 dots | Space + p | 1-2-3-4 + Space |
| Grade 2 Braille Toggle (only for English txt document) | Space + g | 1-2-4-5+ Space |
| Escape | Space +e | 1-5 + Space |
| Begin auto scroll | Space + Right Joystick_Right |  |
| Stop auto scroll | Enter | 8 |
| Increase auto scroll speed | Right Joystick_Right |  |
| Decrease auto scroll speed | Right Joystick Left |  |
| Fast exit to main menu | Space + q | 1-2-3-4-5 + Space |
| Display current time | Space + t | 2-3-4-5 + Space |
| Find | Space + f | 1-2-4+ Space |
| Find Next word | Space+Enter+f, LJ_Up + RJ_Right | 1-2-4+Space+Enter |
| Find Previous word | Space +Backspace+f; LJ_Up + RJ_Left | 1-2-4 + Space+Backspace |
| Set Mark | Space + m | 1-3-4 + Space |
| Delete Mark | Space+Enter+d | 1-4-5-8 + Space |
| Jump to Mark | Space + j | 2-4-5+ Space |
| Jump to Next mark | Space+Entere+j | 2-4-5-8+Space |
| Jump to Previous mark | Space+Backspace+j | 2-4-5-7+Space |
| Go to beginning of previous sentence | RJ_Up | 1 + Space |
| Go to beginning of next sentence | RJ_Down | 4 + Space |
| Go to beginning of current sentence |  | 1-4+ Space |
| Go to beginning of previous paragraph | LJ_Up | 2-3 + Space |
| Go to beginning of next paragraph | LJ_Down | 5-6+ Space |
| Go to beginning of document | Space + LJ_Up | 1-2-3+ Space |
| Go to end of document | Space + LJ_Down | 4-5-6 + Space |

- 2.4 USB connect mode

This application will enable you to use your Mini  Seikaas a PC input/output interface. You will need to connect the USB cable and to launch the screen reader on the PC or Apple device. The Mini  Seika supports JAWS,Window-Eyes, HAL and VoiceOver.

Once this application starts, the Mini  Seika will display “wait connect…”. Then you can start your screen reading software.

If you already set up the connection successfully, there is a simple and fast connect way. After you start the Mini  Seika, please DO NOT touch any keys, connect the USB cable and to launch the screen reader on the PC or Apple device.

- 2.5 Bluetooth mode

Using this application allows you to use the Bluetooth interface of the Mini  Seika. Once this application is started, the Mini  Seika will display “wait connect...”, two seconds later, it will display “btname>tsm ****”, where **** is the Bluetooth name serial number. This is necessary in case there are several Mini  Seikas in one room.

When you are asked to input the Mini  Seika Bluetooth pin, enter “0000” to set up the connection.

If you already set up the connection successfully, there is a simple and fast connect way. After you start the Mini  Seika, please DO NOT touch any keys, launch the screen reader on the PC or cell phone

- 2.6 Clock

This application displays the time and date. There are five sub menus:

2.6.1 Display Current Time

For example, if the time is 40:20:12, Mini Seika displays cl>40h20m12s which is 24 hour time, cl>04h20m12s pm which is 12 hour time. Press Enter or Space +e (1-5 + Space) to escape.

2.6.2 Display Current Date

For example, if the date is 25.06.2012, Saturday, the Mini  Seika will display cl>sat 25d06m12y.Press Enter or Space +e (1-5 + Space) to escape.

2.6.3 Set Time

cl>set time

Press Enter to begin setting the time by changing the hour, minute, second and am/pm. Press Joystick Left and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space +e (1-5 + Space) to cancel the setting.

2.6.4 Set Date

cl>set date

Press Enter to begin setting the date by changing the day, month and year. Press Joystick Left  and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space +e (1-5 + Space) to cancel the setting.

2.6.5 Set hour format

cl>time format

Press Enter to begin time format, the Mini Seika will display “12 24”. You can toggle the 12 or 24 hour format by pressing Joystick Left and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space +e (1-5 + Space) to cancel the setting.

The default time format is 12 hour.

- 2.7 Calculator mode

2.7.1 The basic calculator

With the calculator you can perform arithmetical functions such as addition, subtraction, multiplication, division.

| Operator | Standard Braille | Braille |
| --- | --- | --- |
| Decimal separator | . | 4-6 |
| Addition | + | 3-4-6 |
| Subtraction | - | 3-6 |
| Multiplication | * | 1-6 |
| Division | / | 3-4 |
| Equal | Enter | 8, or Joystick_Center |
| Clear | BackSpace | 7 |

*2.7.2 The scientific calculator

The scientific Calculator allows you to perform more scientific arithmetic operations, such as: exp(x), ln(x), log10(x), sqrt(x), cbrt(x), sin(x), cos(x), tan(x) and pow(x,y), which x or y is the number.

You should input the number and then press Enter to get the result. You should input the left and right bracket if use scientific formula

| Operator | Standard Braille | Braille |
| --- | --- | --- |
| exp(x) calculate e^x, e is napierian base. | exp(operands) | dots15-dots1346-dots1234- dots12356-operands-dots23456 |
| ln(x) natural logarithm, x>0. | ln(operands) | dots123-dots1345- dots12356-operands-dots23456 |
| log10(x) base-10 logarithm, x>0. | log10(operands) | dots123-dots135-dots1245-dot2-dots356- dots12356-operands-dots23456 |
| sqrt(x) square root, x>0. | sqrt(operands) | dots234-dots12345-dots1235-dots2345- dots12356-operands-dots23456 |
| cbrt(x) cube root function. | cbrt(operands) | dots14-dots12-dots1235-dots2345- dots12356-operands-dots23456 |
| sin(x) trigonometric sine function, x is radian system. | sin(operands) | dots234-dots24-dots1345- dots12356-operands-dots23456 |
| cos(x) trigonometric cosine function, x is radian system. | cos(operands) | dots14-dots135-dots234- dots12356-operands-dots23456 |
| tan(x) trigonometric tangent function, x is radian system. | tan(operands) | dots2345-dot1-dots1345- dots12356-operands-dots23456 |
| pow(x) power, calculate x^y, y>0, y is integer. | pow(operands) | dots1234-dots135-dots2456- dots12356-operands-dots23456 |

- 2.8 Tools mode

The tools application allows you access to the information center of your device.

You can press Space+m (1-3-4 + Space) to mute all alarm beep, and press again to enable alarm beep in the Tools and sub-menu. You can press Space+r(1-2-3-5+Space) to restore settings.

2.8.1 Power Menu

tls>power

This shows the power source and battery capacity. If your Mini  Seika is not charging, it will display “not charge xx%”. When the battery voltage is low, the Mini  Seika will alarm three tone every 15 minutes. When you plug in the adapter, the Mini  Seika will beep two tone and display “usb charging”. When the battery is charged fully, the Mini  Seika will beep three tone, and it will display “usb charged full”.

Press Space +e (1-5 + Space) or Enter or Joystick Left  to exit..

2.8.2 Update

tls>update

This feature allows you to update the firmware. You need to download the update BIN file onto the USB disk, and plug it into Mini Seika USB slot. Press Enter and the Mini  Seika will update the firmware, and itt will take about 8 seconds. After the update is complete, it will beep twice and display “pls restart”. Then turn off the Mini  Seika and restart.

Note 1: Do not remove the USB disk or turn the power off when updating the firmware or the Mini  Seika firmware will be corrupted.

Note 2:If the update fails, and the Mini  Seika doesn’t restart, don't worry. Keep pressing Space + u (1-3-6 + Space)and turn the power on. When it  displays “update”, press Enter, and it will begin the update.

2.8.3 Test

tls>test

This feature allows you to test the Mini  Seika braille cells and keys. There are two sub menus:

ts>braille line

This function will move all the pins in sequence. Press Enter to change the display test mode. Press Space +e (1-5 + Space)or Joystick Left  to escape.

ts> keyboard

This function will enable you to test each key. When you press any key and release it, the display pins will change. Press Space +e (1-5 + Space)or Joystick Left  to escape.

2.8.4 About

tls>about

This feature displays the current version of the program, the format is “*.***”, such as “v1.001”. Press Enter or Space +e (1-5 + Space) or Joystick Left  to escape.

2.8.5 Set sleep

tls>set sleep

Press Enter or Joystick_Right to set the sleep waiting time, and the Mini  Seika will display “1 minute”. You can select the sleep waiting time by pressing Joystick_Up and Joystick Down. Such as select 5 minutes, then press Enter to accept the new setting or press Space +e (1-5 + Space)or Joystick Left  to cancel setting.

The default sleep waiting time is 1 minute. You can set the sleep waiting time from 1 minute to 30 minutes. We suggest that you do not set the waiting time more than 5 minutes for extending the braille cells and battery working life time

2.8.6Help

tls>help

This function will show the help document on the Micro SD card. You could use the Read function hot key to read the help document. Press Space +e (1-5 + Space) to escape. If there is no help file in Micro SD card, it will display “no help file”

- *2.9 Transfer file between Mini Seika and computer

This function allows you transfer file between Mini  Seika and your computer. The file size limitation is 1M bytes. You need run the Transfile terminal program in your computer, you can find this program, Transfile.exe, in the CD. Transfile program can show all folders and files in Mini  Seika, so you can use hotkey to copy or paste file between Transfile and computer

If you want to set up the connection, please use USB cable connect Mini  Seika and your computer, then restart Mini  Seika and press Space+c (1-4 + Space) when you are in the root menu of  Mini  Seika. Then, the Mini  Seika will be waiting for the connection. Then run the Transfile in your computer, the connection will be set up. If the connection is not set up automatically , please click the “Connect” button on the Transfile program.

You can press Windows property key to show the file operation list in the Tranfile program file list tree window.

The functions and hot key of Transfile program in your computer:

Alt+F4: Close this program.

Tab: Toggle the focus on the program panel.

Backspace: Go back to the upper directory.

Enter: Open the highlight directory.

Up-Key: Highlight the previous item.

Down-key: Highlight the next item.

Ctrl+c: Copy the file into the computer clipboard

Ctrl+v: Paste the file into the directory.

- 3. Configuring the Bluetooth Connection

This section instructs you how to install and setup Bluetooth on your PC. If there is already Bluetooth built into your computer, please skip section 3.1 and go to section 3.2. If there is no Bluetooth device in your computer, please follow the steps in section 3.1.

- 3.1 Installing the Bluetooth Dongle Driver

Open the “USB Bluetooth dongle driver” folder on the CD. Run the “setup.exe”.

Install the driver using the default steps. The Bluetooth dongle driver will be installed automatically. A console window appears; it takes several minutes.

Click “finish”, and the Bluetooth dongle driver will be installed successfully.

Find the USB Bluetooth dongle that came with your Mini  Seika package and plug it into your computer.

- 3.2 Setting up the Bluetooth Connection

Bluetooth is a short-range wireless communications technology.

The Mini  Seika with Bluetooth capability can communicate with a device over a distance of about 10 meters (30 feet) without requiring a USB cable.

In order to use the Mini  Seika through Bluetooth, you need to establish a partnership with a Bluetooth enabled computer. Most laptops have built-in Bluetooth. If there isn’t built-in Bluetooth on your computer, you need to plug in an external USB Bluetooth dongle (find it in the Mini  Seika) and install the appropriate drivers on your computer.

Here are instructions for setting up the Bluetooth connection between the Mini  Seika and your computer.

Do not connect the USB cable to the computer (use the USB cable and power adapter for charging the Mini  Seika).

Turn on the Mini  Seika and open the Mini  Seika Bluetooth following the steps in section 2.5.

Make sure that Bluetooth is enabled on your computer. If you are using an external USB Bluetooth adapter, make sure you’ve inserted it into your computer.

Double click the Bluetooth icon in the Windows desktop to open Bluetooth Manager, and search for Bluetooth devices. It will take several seconds. When the search is complete, the Braille display Bluetooth name TSM appears in the list of devices.

Select TSM from the list. If the computer doesn’t find the Mini  Seika, please make sure you turn on the Mini  Seika, and then search again.

In the Bluetooth passcode edit field, type 0000.

After a few seconds, you will receive a message that the Bluetooth device was successfully connected and the outgoing and incoming COM port assignments are displayed. Make a note of the outgoing port number as this is the port the screen reader will use to communicate with the Mini  Seika.

Now you are ready to configure your screen reader to use the Bluetooth COM port. To configure the COM port in the screen reader, please refer to chapter 4 for Jaws, chapter 5 for Window-Eyes.

- 4. Using the Mini  Seika with JAWS

- 4.1 Installing the Seika Patch for JAWS

Open the “Patch for JAWS” folder in the CD. Run the Patch for JAWS according to your Windows operating system.

Install the patch by using the default steps. The Patch for JAWS will be installed by the JAWS Utility automatically. A console window appears which will take several minutes.

- 4.2 Working in JAWS

4.2.1 Configuring JAWS

Start JAWS and select “Braille…” in the “Options” menu.

Assuming that the default Braille display is set to “No display”, tab to the “Add Braille Display” button and press Enter.

In the “Select components” dialog box, there is a list of Braille displays. Select the “Seika” by pressing the Space bar.

Tab to the “Next” button and press Enter.  If you connected the Mini  Seika to your computer by USB cable, select “USB” as the output port.

If you connected the Mini  Seika to a PC using Bluetooth, in the “Set device options” dialog box, select the output port that corresponds to the COM port number that you made note of in chapter 3.2.Tab to the “Next” button and press enter.

When prompted to set the primary device, select “Seika” in the “Primary device” list.

Tab to the “Finish” button and press Enter.

In the “Braille Basic Settings” dialog box, tab to the “OK” button and press it.

Restart JAWS and the Mini  Seika will now work .

4.2.2 Navigation keys and commands

| Function | Keys |
| --- | --- |
| Enter | 9A |
| Enter | RJ_Center |
| Space | A |
| JAWS Back Space | 9 |
| Up Arrow Key | RJ_Up |
| Down Arrow Key | RJ_Down |
| Left Arrow Key | RJ_Left |
| Right Arrow Key | RJ_Right |
| Braille Pan Right | RB |
| Braille Pan Left | LB |
| Stop Speech | 9+RB |
| Braille Moves Active | LJ_Left+RJ_Up |
| Active Moves Braille | LJ_Left+RJ_Down |
| Braille Toggle Marking | LJ_Center+RJ_Up |
| Braille Next Line | LJ_Right |
| Braille Prior Line | LJ_Left |
| Start JAWS Task List | LJ_Right+RJ_Up |
| List Task Tray Icons | LJ_Right+RJ_Dowm |
| Right Mouse Button | LJ_Right+RJ_Right |
| Left Mouse Button | LJ_Right+RJ_Left |
| Left Mouse Button Lock | A+LJ_Right |
| Route JAWS Cursor To Pc | LJ_Right+RJ_Center |
| Route PC Cursor To JAWS | LJ_Left+RJ_Center |
| Braille Bottom | LJ_Down |
| Braille Top | LJ_Up |
| Mini Seikamize AllApps | 9+LJ_Center |
| Select A Link | LJ_Down+RJ_Left |
| Select A Frame | LJ_Up_Up+RJ_Down |
| Select A Heading | LJ_Up+RJ_Left |
| Select A Form Field | LJ_Up+RJ_Right |
| Braille Toggle 8 Dots | A+LB |
| Bottom Of File | LJ_Up+RJ_Up |
| Top Of File | LJ_Down+RJ_Down |
| PC Cursor | RB+LJ_Right |
| JAWS Cursor | RB+LJ_Left |
| Braille Toggle Mode | LB+RB |
| Braille Toggle 8 Pixels Per Space | A+RB |
| Braille Toggle Characters And Attributes | LJ_Left+RJ_Left |
| Braille Turn Marking Off | 9+LB |
| Increase Braille Auto Advance Speed | LB+RJ_Center |
| Decrease Braille Auto Advance Speed | LB+LJ_Center |
| JAWS Page Down | LJ_Down+RJ_Center |
| JAWS Page Up | LJ_Up+RJ_Center |
| Braille Tab | LJ_Center+RJ_Right |
| Braille Shift Tab | LJ_Center+RJ_Left |
| Next Document Window | RB+LJ_Down |
| Previous Document Window | RB+LJ_Up |
| Route Braille To Active Cursor | RB+LJ_Center |
| Braille Select Text | RJ_Center+Routing |
| Repeat Last Flash Message | LJ_Down+RJ_Up |
| Braille Auto Advance | LJ_Down+RJ_Right |

- 4.3 Uninstalling the patch for JAWS

Note: If you uninstall the JAWS patch, it will disable the Mini  Seika’s ability to work with JAWS and delete all the associated patch files. You must uninstall this patch first before uninstalling JAWS.

4.3.1 Uninstalling the old version of the Seika patch for Jaws

Open the “Add or Remove Programs” in the “Control Panel”, and select the item: “Seika Braille Display version 3” or “Seika Patch for JAWS V1.x” or “Seika Patch for JAWS V2.x”. Uninstall the item step by step. The uninstall will take several minutes. Please restart the computer after the uninstall.

4.3.2 Uninstall the current version of the Seika patch for Jaws

Open the “Add or Remove Programs” in the “Control Panel”, and select the item: “Seika Patch for JAWS V2.3”. Uninstall the “Seika Patch for JAWS V2.3” step by step. The uninstall will take several minutes. Please restart the computer after the uninstall finishes.

- 5. Using the Mini  Seika with Window-Eyes

- 5.1 Configuring Window-Eyes 7.5.2

Note: We strongly recommend you update Window-Eyes to version 7.5.2 or later.

If you connect the Mini  Seika to your computer using Bluetooth, in the “Set device options” dialog box, select the output port that corresponds to the COM port number that you made note of in chapter 3.

Start Window-Eyes 7.5 and select “Device” in the “Settings” list.

Select “Braille Display” in the “Device” list.

In the “Braille Displays” option, select “Seika”.

In the “Ports” option, if you connect the Mini  Seika to your computer using USB, select the USB connection.

If you connect the Mini  Seika to your computer using Bluetooth, select the Seika COM port number that you made note of in chapter 3.

Tab to the “Activate” button and press Enter.

Select “Keep Changes” button in the “Braille Settings” dialog box, then press Enter.

Tab to “Save Settings” button and press Enter.

- 5.2 Navigation Keys and Commands

| Bottom Line | RJ_Right |
| --- | --- |
| Mouse Bottom Left | LB+RJ_Down |
| Mouse Bottom Right | RB+LJ_Right |
| Mouse Top Left | LB+RJ_Up |
| Mouse Top Right | RB+LJ_Left |
| To Focus | LB+RB |
| 6/8 Dot Toggle | Space +LB |
| Untranslate at Cursor | LJ_Center+RJ_Left |
| Untranslate at Mouse | LJ_Center+RJ_Right |
| Braille Graphic Mode | LJ_Center+RJ_Down |
| Scroll Mode Toggle | LJ_Up+RJ_Left |
| Show Attributes Toggle | LJ_Down+RJ_Center |
| Grade 2 Braille Toggle | LJ_Center+RJ_Center |
| Braille Tracking Toggle | LJ_Down+RJ_Down |
| Spacing Toggle | LJ_Down+RJ_Right |
| Attributes To Show Rotor | LJ_Up+RJ_Center |

| Display Individual Attribute Rotor | LJ_Center+RJ_Up |
| --- | --- |
| Quick Message Rotor | LJ_Down+RJ_Left |

- 6. Using the Mini  Seika with VoiceOver

- 6.1 Using VoiceOver on the iPad and iPhone

Do not connect the USB cable with your computer (use the USB cable and power adapter for charging the Mini  Seika). Turn on the Mini  Seika and enable Mini  Seika Bluetooth following the steps in section 2.5. Once Mini  Seika Bluetooth is started, it will display “wait connect...”, two seconds later, it will display “btname>tsm ****”, where **** is the Bluetooth name serial number.

Make sure your iPhone Bluetooth is turned off.

Enable the voiceover by the following steps: iPhone Settings - General - Accessibility – VoiceOver.

Enalbe the Braille by the following steps: VoiceOver – Braille. When you are asked whether open Bluetooth or not, please select open Bluetooth, then iPhone will search the Mini  Seika, and it will take about one minutes. When iPhone find Mini  Seika and display Bluetooth name TSM ****, you need double click this item and input the pin code 0000 to set up the connection.

If you already set up the connection successfully one time, there is a simple and fast connect way when you need set up the the connection once again. After you start the Mini  Seika, please DO NOT touch any keys, and launch the voiceover.

- 6.2 Using VoiceOver on a Mac OS X computer

Do not connect the USB cable with your computer (use the USB cable and power adapter for charging the Mini  Seika).

Turn on the Mini  Seika and open the Mini  Seika Bluetooth following the steps in section 2.5.

Launch VoiceOver Utility

Click on the "Braille" tab on the left.

Click on the "Displays" tab.

The Braille display Bluetooth name will appear in the list of Braille devices. Its name will begin with TSM in the list of devices.

Click the device name to select it, and initiate the pairing.

You will be taken to a passcode entry screen, enter the passcode 0000.

- 6.3 Navigation keys and commands

Note: When using a Braille display with VoiceOver using IOS 6 or later, you can use the following key commands to navigate.

Common Braille Commands for VoiceOver Navigation:

| Function | keys |
| --- | --- |
| Move to previous item | 1 + Space |
| Move to next item | 4 + Space |
| Move to previous item using rotor setting | 3 + Space |
| Move to next item using rotor setting | 6 + Space |
| Select previous rotor setting | 2-3 + Space |
| Select next rotor setting | 5-6 + Space |
| Move to the first element | 1-2-3 + Space |
| Move to the last element | 4-5-6+ Space |
| Read page starting at selected item | 1-2-3-5 + Space |
| Read page starting at the top | 2-4-5-6+ Space |
| Activates the Home button | 1-2-5 + Space |
| Goes to the Status bar | 2-3-4+ Space |
| Activates the Volume Up button | 2-4-5 + Space |
| Activates the Volume Down button | 1-2-6 + Space |
| Activates a Back button if present | 1-2 + Space |
| Activates the Eject key | 3-4 + Space |
| Activates the Delete key | 7 + Space |
| Activates the Delete key | 1-4-5 + Space |
| Activates the Return (Enter) key | 8 + Space |
| Activates the Return (Enter) key | 1-5 + Space |
| Activates the Tab key | 2-3-4-5+ Space |
| Toggle Screen Curtain on and off | 1-2-3-4-5-6 + Space |
| Pause or Continue speech | 1-2-3-4+ Space |
| Toggle speech on and off | 1-3-4 + Space |
| Scroll left one page | 2-4-6+ Space |
| Scroll right one page | 1-3-5 + Space |
| Deselect text | 2-3-6 + Space |
| Select text | 3-5-6 + Space |
| Pan Braille to the left | 2 + Space |
| Pan Braille to the right | 5 + Space |
| Toggle announcement history | 1-3-4-5 + Space |
| Switch between contracted and uncontracted Braille | 1-2-4-5 + Space |

Device keys for VoiceOver navigation:

| Function | keys |
| --- | --- |
| Pan braille to the left | LB |
| Pan braille to the right | RB |
| Select item under your finger | LJ_Center |
| Move to previous item | LJ_Left |
| Move to next item | LJ_Right |
| Move to previous item using rotor setting | LJ_Up |
| Move to next item using rotor setting | LJ_Down |
| Select item under your finger | RJ_Center |
| Move to previous item | RJ_Left |
| Move to next item | RJ_Right |
| Move to previous item using rotor setting | RJ_Up |
| Move to next item using rotor setting | RJ_Down |
| Double tap the selected item | Router |
| Toggle expanded status description | Status Router |

- 7. Using the Mini  Seika with NVDA

- 7.1 Pairing Braille Devices

After you run NVDA, select Seika Notetaker in the braille display list.

- 7.2 Braille Commands

| Function | Keys |
| --- | --- |
| braille_scroll Back | LB |
| braille_scroll Forward | RB |
| braille_previous Line | LJ_Up |
| braille_next Line | LJ_Down |
| braille_toggle Te ther | LJ_Center |
| Say All | Space+BackSpace |
| shift+tab | Dot2 |
| Show Gui | RB+LB |
| tab | LJ_Right |
| shift+tab | LJ_Left |
| Up Arrow | RJ_Up |
| Down Arrow | RJ_Down |
| Left Arrow | RJ_Left |
| Right Arrow | RJ_Right |
| enter | RJ_Center |
| shift+up Arrow | Space+RJ_Up |
| shift+down Arrow | Space+RJ_Down |
| shift+left Arrow | Space+RJ_Left |
| shift+right Arrow | Space+RJ_Right |
| escape | Space+RJ_Center |
| shift+up Arrow | BackSpace+RJ_Up |
| shift+down Arrow | BackSpace+RJ_Down |
| shift+left Arrow | BackSpace+RJ_Left |
| shift+right Arrow | BackSpace+RJ_Right |
| windows | BackSpace+RJ_Center |

- 8. Using the Mini Seika with Mobile Accessibility v2.0 for Android

- 8.1 Pairing Braille Devices

Do not connect the USB cable with your computer (if you will charge the Mini  Seika, please use power adapter).

Turn on the Mini  Seika and open the Mini  Seika Bluetooth following the steps in section 2.5.Start your braille device, check the device manual for instructions to start and enable Bluetooth on the device.

Go to Mobile Accessibility > Apps > Settings > Wireless and Network > Bluetooth Settings.

Check the option “Bluetooth”.

In the same window use, “Scan Devices” to let the device scan nearby Bluetooth devices.

After few seconds, Scroll in the window to find your Braille display device.

It will prompt you to enter passcode / pin for the device, enter the passcode “0000”

Enter the pin of the device, for pin information for pairing, check the device user manual.

- 8.2 Configuring Braille Display with Mobile Accessibility

Go to Mobile Accessibility > Settings > Braille Settings.

Activate the option “Braille Device”

In the list select the Braille device you wish to use. This list contains all Bluetooth devices paired with the phone.

Stay in the braille settings and activate the option “Device Model”

In the list, select the model of Seika Notetaker.

Exit the Braille settings and the setting window.

Congratulations, the braille device is configured.

- 8.3 Removing Braille Display configuration

Follow the below steps

Go to Mobile Accessibility > Apps > Settings > Wireless and Network > Bluetooth Settings.

Check the option “Bluetooth”.

In the same window, scroll down to find the device you wish to remove.

Hit Long enter on the device and select “Unpair”

- 8.4 Enabling / Disabling Braille

To enable or to start braille with Mobile Accessibility:

Activate quick menu, use the menu key or triple tap the screen within Mobile Accessibility window.

Select the option “Enable Braille”.

Mobile Accessibility will beep and will announce “Braille Ready”.

To disable or to stop Braille with Mobile Accessibility:

Activate quick menu, use the menu key or triple tap the screen within Mobile Accessibility window.

Select the option “Disable Braille”.

Mobile Accessibility will beep and will announce “Braille off”

- 8.5 Braille Commands

Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8.

Cursor-routing is above on every Braille cell.

Left button and Right button are on the left and right side of the Braille cell area respectively.

Left Joystick and Right Joystick are on the below of the Left button and Right button respectively. Left Joystick Left (LJ_Left), Left Joystick Right (LJ_Right), Left Joystick Previous (LJ_Up), Left Joystick Next (LJ_Down) and Left Joystick Center (LJ_Center) are the Left Joystick 5 keys. The Right Joystick also has 5 keys: RJ_Left, RJ_Right, RJ_Up, RJ_Down and RJ_Center.

Dot7 and Dot8 represent BackSpace and Enter respectively if Dot7 and Dot8 aren’t use for typing.

There are two Space Buttons between the Left Joystick and Right Joystick.

| Function | Keys |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter,RJ_Center,LJ_Center, Cursor-Routing in Braille Cursor mode |
| Home | Space + BackSpace + dots125(h) |
| Back(Escape) | Space + dots12(b) |
| Insert Space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Braille BackSpace | BackSpace |
| Clear/Delete | Space + dots145(d) |
| Braille Pan Left | Left |
| Braille Pan Right | Right |
| Previous Character/Grid Item/Tab Sheet | LJ_Left,RJ_Left,Space + dot3 |
| Next Character/Grid Item/Tab Sheet | LJ_Right,RJ_Right,Space + dot6 |
| Previous Line/List Item/Menu Item | LJ_Up,RJ_Up,Space + dot1 |
| Next Line/List Item/Menu Item | LJ_Down,RJ_Down,Space + dot4 |
| Previous Word | Space+LJ_Left,Space+RJ_Left, Space+dot2 |
| Next Word | Space +LJ_Right, Space + RJ_Right,Space + dot5 |
| Left Softkey | Space + dot7 |
| Right Softkey | Space + dot8 |
| Dial/Send | Space + BackSpace + dot3 |
| Hang-Up/End Call | Space + BackSpace + dot6 |
| Application Menu (Switch Application) | LJ_Center+RJ_Center, Space+ dots134(m) |
| List of Open Applications(Show Start Menu) | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots24(i) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile (Switch Profile) | Space + dots1234(p) |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + BackSpace + dots1236(v) |
| Increase Volume | Space + Enter + dots1236(v) |
| Decrease Speed | Space +BackSpace + dots234(s) |
| Increase Speed | Space+ Enter + dots234(s) |
| Change Verbosity Level | Space + dots1236(v) |
| Change Keyboard Echo | Space + Enter + dots15(e) |
| Read Date and Time | Space + dots2345(t) |
| Toggle Speech Mute | Space + BackSpace + dots136(u) |
| Interrupt/Stop Speech | Space + BackSpace + dots24(i) |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor (Toggle Tactile Cursor) | Space + dots123456 |
| Braille Routing | Routing |

- Using the Mini  Seika with Mobile Speak for Symbian

| Function | Keys |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter,RJ_Center,LJ_Center, Cursor-Routing in Braille Cursor mode |
| Home | Space + BackSpace + dots125(h) |
| Back(Escape) | Space + dots12(b) |
| Insert Space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Braille BackSpace | BackSpace |
| Clear/Delete | Space + dots145(d) |
| Braille Pan Left | Left |
| Braille Pan Right | Right |
| Previous Character/Grid Item/Tab Sheet | LJ_Left,RJ_Left,Space + dot3 |
| Next Character/Grid Item/Tab Sheet | LJ_Right,RJ_Right,Space + dot6 |
| Previous Line/List Item/Menu Item | LJ_Up,RJ_Up,Space + dot1 |
| Next Line/List Item/Menu Item | LJ_Down,RJ_Down,Space + dot4 |
| Previous Word | Space+LJ_Left,Space+RJ_Left, Space+dot2 |
| Next Word | Space +LJ_Right, Space + RJ_Right,Space + dot5 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Space + BackSpace + dot3 |
| Hang-Up/End Call | Space + BackSpace + dot6 |
| Application Menu (Switch Application) | LJ_Center+RJ_Center, Space+ dots134(m) |
| List of Open Applications(Show Start Menu) | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots24(i) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile (Switch Profile) | Space + dots1234(p) |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + BackSpace + dots1236(v) |
| Increase Volume | Space + Enter + dots1236(v) |
| Decrease Speed | Space +BackSpace + dots234(s) |
| Increase Speed | Space+ Enter + dots234(s) |
| Change Verbosity Level | Space + dots1236(v) |
| Change Keyboard Echo | Space + Enter + dots15(e) |
| Read Date and Time | Space + dots2345(t) |
| Toggle Speech Mute | Space + BackSpace + dots136(u) |
| Interrupt/Stop Speech | Space + BackSpace + dots24(i) |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor (Toggle Tactile Cursor) | Space + dots123456 |
| Braille Routing | Routing |

- 10. Using the Mini Seika with TALKS

| Function | Keys |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter,RJ_Center,LJ_Center, Cursor-Routing in Braille Cursor mode |
| Home | Space + BackSpace + dots125(h) |
| Back(Escape) | Space + dots12(b) |
| Insert Space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Braille BackSpace | BackSpace |
| Clear/Delete | Space + dots145(d) |
| Braille Pan Left | LB |
| Braille Pan Right | RB |
| Previous Character/Grid Item/Tab Sheet | LJ_Left,RJ_Left,Space + dot3 |
| Next Character/Grid Item/Tab Sheet | LJ_Right,RJ_Right,Space + dot6 |
| Previous Line/List Item/Menu Item | LJ_Up,RJ_Up,Space + dot1 |
| Next Line/List Item/Menu Item | LJ_Down,RJ_Down,Space + dot4 |
| Previous Word | Space+LJ_Left,Space+RJ_Left, Space+dot2 |
| Next Word | Space +LJ_Right, Space + RJ_Right,Space + dot5 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Space + BackSpace + dot3 |
| Hang-Up/End Call | Space + BackSpace + dot6 |
| Application Menu (Switch Application) | LJ_Center+RJ_Center, Space+ dots134(m) |
| List of Open Applications(Show Start Menu) | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots24(i) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile (Switch Profile) | Space + dots1234(p) |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + BackSpace + dots1236(v) |
| Increase Volume | Space + Enter + dots1236(v) |
| Decrease Speed | Space +BackSpace + dots234(s) |
| Increase Speed | Space+ Enter + dots234(s) |
| Change Verbosity Level | Space + dots1236(v) |
| Change Keyboard Echo | Space + Enter + dots15(e) |
| Read Date and Time | Space + dots2345(t) |
| Toggle Speech Mute | Space + BackSpace + dots136(u) |
| Interrupt/Stop Speech | Space + BackSpace + dots24(i) |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor (Toggle Tactile Cursor) | Space + dots123456 |
| Braille Routing | Routing |

- 11. Using the Mini Seika with android Talkback

Your phone must be Android 4.1 or above it, and please download and install Talkback and Braille Back.

Do not connect the USB cable with your computer (if you will charge the Mini Seika, please use power adapter).

Firstly, turn on the Mini  Seika and open the Mini Seika Bluetooth following the steps in section 2.5

Secondly, turn on your phone Bluetooth and search the Bluetooth device of TSM **** (where **** means Mini  Seika Bluetooth serial number). If need, please enter the pin code “0000”.

| Function | Keys |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter,RJ_Center,LJ_Center, Cursor-Routing in Braille Cursor mode |
| Activate the main screen | BackSpace + dots125(h) |
| Back(Escape) | Space + dots12(b) |
| Insert Space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Braille BackSpace | BackSpace |
| Clear/Delete | Dots7 |
| Braille Pan Left | LB |
| Braille Pan Right | RB |
| Previous Character/Grid Item/Tab Sheet | LJ_Left,RJ_Left,Space + dot3 |
| Next Character/Grid Item/Tab Sheet | LJ_Right,RJ_Right,Space + dot6 |
| Previous Line/List Item/Menu Item | LJ_Up,RJ_Up,Space + dot1 |
| Next Line/List Item/Menu Item | LJ_Down,RJ_Down,Space + dot4 |
| Previous Word | Space+LJ_Left,Space+RJ_Left, Space+dot2 |
| Next Word | Space +LJ_Right, Space + RJ_Right,Space + dot5 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Space + BackSpace + dot3 |
| Hang-Up/End Call | Space + BackSpace + dot6 |
| Application Menu (Switch Application) | LJ_Center+RJ_Center, Space+ dots134(m) |
| List of Open Applications(Show Start Menu) | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots24(i) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile (Switch Profile) | Space + dots1234(p) |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + BackSpace + dots1236(v) |
| Increase Volume | Space + Enter + dots1236(v) |
| Decrease Speed | Space +BackSpace + dots234(s) |
| Increase Speed | Space+ Enter + dots234(s) |
| Change Verbosity Level | Space + dots1236(v) |
| Change Keyboard Echo | Space + Enter + dots15(e) |
| Read Date and Time | Space + dots2345(t) |
| Toggle Speech Mute | Space + BackSpace + dots136(u) |
| Interrupt/Stop Speech | Space + BackSpace + dots24(i) |
| Toggle Command Help Mode | Space + dots125(h) |
| Braille Routing | Routing |