Mini Seika

User’s Manual

Version 1.14

2012 August

Preface

Thank you very much for purchasing Mini Seika!

The Mini Seika is a portable 16 cells Braille display with integrated Braille keyboard.

Before using the Mini Seika, please read this user’s manual.

If there is anything that you don’t understand about using the display, or if you find mistakes or omissions in this user’s guide, please contact distributors by letter, fax, or telephone.

Note:

Unauthorized reproduction of this document, in whole or in part, is strictly prohibited.

Modifications to the contents of this document for improvements or revisions may be made without prior notification.

Product liability is limited within the country of original purchase.

Table of Contents

1.1 Equipment Checklist	1

1 Getting Start	1

# 1 Getting Start

Welcome to the Mini Seika, an innovative personal productivity system. Mini Seika is a portable 16 cells Braille display with note taking and a suite of powerful applications. Mini Seika is compatible with several popular screen reading software for mobile phones and PC, so it will work well in conjunction with a mobile phone and PC via Bluetooth or USB cable. This user guide tells you all about them.

Mini Seika includes the following applications: PC connection via USB or Bluetooth, connection to a mobile phone via Bluetooth, Notepad, File Manage, Read, Clock, and Calculator.

## 1.1 Equipment Checklist

Carefully unpack your Mini Seika. Save the box and packing materials for future use. Your Mini Seika package comprises the following items:

Mini Seika in the leather carry case

Slim line tape for carry case (you can lock the tape two head with each ring on the each side of the carry case)

Power adapter which output is 5V/1A

USB Cable (one head is USB male, the other head is mini USB male), it is for battery charging and screen reader data transfer

4G Micro SD card which is in the Mini Seika in default

Bluetooth USB dongle

USB disk

Documentation and Drivers on a CD

User’s Manual

## 1.2 Device description

Mini Seika total weight is 300g, with dimensions: 6.3 x 3.78 x 0.9 inches.

Mini Seika can operate from its internal rechargeable battery or from its AC adapter via USB cable.

Mini Seika is supplied in a leather carry case. To open the case, lift the flap on the rear edge. Fold back the case to allow operation of the Mini Seika.

Place the Mini Seika on a flat surface with the Braille display near you.

On the left side, from back to front, you will find:

A vertical USB slot which is for the USB disk

A horizontal slot for Micro SD card. The Micro SD card is in the slot of Mini Seika in default. Plugging/unplugging the Micro SD card should be carried out gently. The Micro SD card slot has auto-locking equipment, you just need to press, and then release it in order to unlock the card.

Moreover, you should not unplug the Micro SD card while the device is working, because some files may be opened and could therefore be damaged. Before unplug the Micro SD card, you need close the file you’ve opened in the notepad, and switch off the Mini Seika.

On the right side, from back to front, you will find:

A mini USB interface

A rocker type power switch enabling to power on/off Mini Seika. To switch the Mini Seika on, press the power switch near the front of the rocker.

On the top surface of the Mini Seika:

Mini Seika is equipped with 16 cell (8 dots) piezo-electrical Braille display.

If you run your finger up past braille cells, you will notice that there is a raised vertical ridge through the middle. This is the touch cursor button for that braille cell. It is especially useful when editing a document. In notepad, for example, when you press a touch cursor button, the cursor moves to that cell. You can then edit the character displaying on that cell. The touch cursor can also be used to select an item from a menu or list.

Behind 16 cells, there are eight Braille keys, named 1 to 8.

In the front of 16 cells, there are two keys, the left key is named 9 (Space), the right key is named A (Space).

Mini Seika comes with two 5 Navigation joysticks keys (left, right, up, down and center). Each of them will be called “LJ” for the left joystick key and “RJ” for the right one in this manual.

Mini Seika possesses 2 Navigation keys. Near each end of the Braille display is a button. The Left Button (LB) is to the left and the Right Button (RB) is to the right.

## 1.3 Power Switch and Battery

From the front edge of your Mini Seika, move your hand along the right side of the case and towards the rear, you will come upon a rocker type power switch surrounded by a concave rectangular frame. To switch the Mini Seika on, press the power switch near the front of the rocker. The Mini Seika will immediately beeps twice and shows the “notepad” which is the first function item in the root main menu. To switch off, press the power switch near the rear of the rocker.

If you don’t use the Mini Seika for approximately 1 minute it will enter sleep mode. Pressing any key will end the sleep mode, but it is recommended that you momentarily press the Joystick left or Joystick right to end the sleep mode. This will avoid the possibility of inserting unwanted characters if a document was open when the unit entered the sleep mode. If you don’t use the Mini Seika for approximately 1 hour it will be closed to save power, please note that you CAN NOT wake it up by press any key. The document was open will be saved automatically. The document which is new and not given a name will be saved as a given name “normal” in the root folder. To continue using the Mini Seika, you need switch Mini Seika off, and then you can switch Mini Seika on.

The Mini Seika can operate from its internal rechargeable battery or from its AC adapter. The length of time you can run the Mini Seika from a fully charged battery is typically over 10 hours before it becomes flat. Activities like using USB disk or Bluetooth increase the drain on the battery.

If your Mini Seika displays “battery low” and beeps the alarm, or does not come to life if the switch is on, it means that the battery is completely discharged; in that case, you should plug in the AC adapter. The power socket is the mini USB socket, which is located at the right side, behind the power switch. Plug the smaller end of the USB cable your Mini Seika, and the larger end into the USB port on the AC adapter. Plug the AC adapter into a wall socket, you can hear a hint beep which indicates the battery charge process begins. And you can also connect your Mini Seika with PC via USB cable to charge the battery.

If the battery is flat and the AC adapter is connected, the battery is fast charged in about 4 hours. Charging takes less time if the battery is only partially discharged. You can charge the battery and use the Mini Seika simultaneously although the battery may take longer to charge. Please ensure that the battery charges correctly, you can only charge in ambient temperatures between 41 and 95 degrees Fahrenheit, that is, 5 to 35 degrees Celsius.

## 1.4 Hardware Specifications

Memory extension: Micro SD Card and U-disk.

CPU: ARM

Battery: Lithium rechargeable battery

Keyboard: 8-dots Braille keyboard, 4 function keys, 16 cursor routing keys, 2 Navigation joysticks keys

Braille display:  16 refreshable 8-dots Braille

Wireless: Bluetooth

Interface: USB, MiniUSB, Micro SD card slot

# 2 Basic Functions of the Mini Seika

### 2.0.1 How to Use the Commands

All of the commands that you will use on the Mini Seika are comprised of different combinations of the Braille keys. In this user manual, the “+” sign is used to indicate key combinations. For example, “Space + z” indicates that the user should press the space key and letter z braille (dots 1-3-5-6) simultaneously and then release them at the same time. You will also find expressions like “1356A” in this manual, it is the same meaning. You can execute most commands by pressing the “Enter” button (press Joystick_Center, or press dot 8). If you would like to cancel the execution of any command, or return to the previous status, press “Space + e” (15 + Space).

### 2.0.2 General Functions

Upon switch on the unit, it will beep twice, and on the braille display the first item in the root menu appears: notepad

The root menu includes: notepad, file manage, read, USB connect, Bluetooth, clock, calculator, tools. You can use the key Joystick_Up and Joystick_Down to scroll items in the root menu and press Enter or Joystick_Right or cursor routing keys above the item to select it. You can type the first letter to jump to the corresponding item, for example, you can press the letter “r” (dots 1-2-3-5) to jump to the “read” item.

Note: As the clock and calculator menu have the same first letter, you can type letter “a” to jump to calculator item.

Several root menu items (father item) have the infant items. For example, if you enter the root menu items “tools”, the tools item will appear in an abbreviated word as a father item (tls) with the infant item displayed after the punctuation sign >, as follows:

tls>power

You can enter power infant item by press Enter or Joystick_Right or cursor routing keys above the infant item (for example: press the cursor routing keys above the ‘power’). You can escape by Joystick_Left or press Space + e (15 + Space) or press the cursor routing keys above the father item word (for example: press the cursor routing keys above the ‘tls’).

Trees are used to display the structure of folders and files on the Micro SD card. The file’s name begins with letter f, and the folder’s name begins with letter d. If you press Enter key or Joystick_Right on the folder, it will display the infant tree of the folder. If you press Enter key on the file, it will open the file. If you want to move back up one one level in the tree structure, you can press the Backspace (7) or Space + e (15 + Space) or Joystick_Left.

Mini Seika support several countries language and braille tables. You can switch between different language and braille table by press Space + l (123 + Space) in the root menu. You could use Joystick_Up and Joystick_Down to scroll between different languages, such as “language:usa”, “Language:en”, “lingua:italiano”, “langue:fran?aise”, “اللغة:عربي” and press Enter to select it. You can also press Space + e (15 + Space) to cancel switch language and braille table.

| Action | Hot key | Braille |
| --- | --- | --- |
| switch language and braille table | Space + l | 123 + Space |

## 2.1 Notepad

The notepad program will enable you to edit simple file, each file size can’t exceed 2M. Notepad supports long file name and multi-level folder. If the file name is too long to display at one time, you can use Left Button and Right Button to display the whole file name, file size, modification date and time. All files name are displayed and edited using 6-dot.

In order to use the notepad, you will need to have a Micro SD card in the card slot. In default, there is already a Micro SD card in the card slot of mini Seika. When you start the notepad program, mini Seika will display the first file name in Micro SD card, such as help_en.txt which file in Micro SD card in default, you can use Joystick_Up and Joystick_Down key to scroll the files name and press Enter or Joystick_Right to select it, you can also press backspace or Joystick_Left to move back up one level or press Space + e (15 + Space) to return notepad main menu. If there is no file in Micro SD card, mini Seika will display “no content”. If there is no Micro SD card, it will display “no sd card”, press Enter key or Space + e (15 + Space) to go back to the "notepad" root menu.

All braille files (brl, brf and so on) are displayed and edited using 6-dot. The txt file is displayed and edited using 8-dot in default, and you could press Space + p (1234 + Space) to switch between 6-dot and 8-dot.

Temporary files will be created on the Micro SD card as you edit your document; as long as you do not save your document, additional temporary files will be created; we recommend that you save your document frequently enough so that the creation of those temporary files does not end up slowing down your system.

2.1.1 New File (Space + n; 1345 + Space)

This function will close any open document and start a new document which you can immediately edit. The document which you do not save will be saved as the file name “normalxx” in the root folder, “xx” means the normal file serial number.

2.1.2 Close File (Space + e; 15 +Space)

This function will close any opened document.

If you were on an opened document, press Space + e (15 +Space), you will be asked to save that document. The mini will display “save file? y n c” (where y means save, n means not save and quit, c means cancel and go back to continue edit current file). You can press Joystick_Left, Joystick_Right to move the cursor to y n c and press Enter key, or you can input letter y or n or c to complete the operation.

2.1.3 Save File (Space + s; 234 + Space)

This function enables you to save any modifications you have made in a document. This will save the document under its current name; if you have not yet given the document a name, you will be asked to do so.

Press Space + s (234 + Space) to save the file in the current folder, Mini will ask you input the new name, After you input the new file name, and press Enter key, you will go back to continue to edit the current file. You can also press Space + e (15 + Space) to cancel input file name and go back to continue to edit the current file.

If you want to save a new file under a different folder, please choose folder before save, To do this, in edit file mode, you can press Space + Enter + f (1248 + Space) to go to files and folders list tree (when you power on the Mini, the default working folder is in the root directory). In the tree, you can press backspace to return Upper-level directory, you can also press Space + n (1345 + Space) to create a new folder. You can press Space + e (15 + Space) to confirm the current name and go back to continue to edit file.

Note: The file name should not be longer than 8 characters, and the extend file name should not be longer than 3 characters. The file name and extend file name is separated by dot.

2.1.4 Save As (Space +Enter + s; 2348 + Space)

This function will let you save the document that is currently open in a given file and with a given name.

Press Space +Enter + s (2348 + Space) to save the file in the current folder, Mini will ask you input the new name, After you input the new file name, and press Enter key, you will go back to continue to edit the current file. You can press Space + e (15 + Space) to cancel input file name and go back to continue to edit current file.

If you want to save as the file under a different folder, please choose folder before save as, To do this, in edit file mode, you can press Space + Enter + f (1248 + Space) to go to files and folders list tree (when you power on the Mini, the default working folder is in the root directory). In the tree, you can press backspace to return Upper-level directory, you can also press Space + n (1345 + Space) to create a new folder. You can press Space + e (15 + Space) to continue to edit file.

Note: The file name should not be longer than 8 characters, and the extend file name should not be longer than 3 characters. The file name and extend file name is separated by dot.

2.1.5 Fast Quit (Space + q, 12345 + Space)

This function will close any open document and go back to the root menu. The document which you do not save will be saved as the file name “normalxx” in the root folder, “xx” means the normal file serial number.

2.1.6 Find (Space + f, 124 + Space)

This function allows you to search backwards or forwards for a letter-sequence, starting from the cursor location. To do this, Press Space + f (124 + Space), then type in the letter-sequence (for example: mini) and press Enter to start a search document.

2.1.7 Find Next (Space + x, 1346 + Space)

This will trigger a search (with the same word which you input in the Find function) towards the end of the document.

2.1.8 Find Previous (Space + y, 13456 + Space)

This will trigger a search (with the same word which you input in the Find function) towards the begin of the document.

2.1.9 Set Mark (Space +m, 134 + Space)

This functions marks the line of the document at which the cursor is located. This will enable you to travel back to this line by using the “Jump to Mark” function, even after you have closed and re-opened the document. The mark position is kept in a hidden file on the Micro SD card.

2.1.10 Jump to Mark (Space + j, 245 + Space)

This function allows you to go to the mark position.

Note: Do not pull out the Micro SD card when you are editing file. If you need pull out the Micro SD card, please turn power off.

2.1.11 Braille and hot key summary

Manu Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Space |  | 9, A, 9A |
| Backspace |  | 7 |
| Enter | Joystick_Center | 8 |
| Escape | Space + e | 15 + Space |
| Fast exit to main menu | Space + q | 12345 + Space |
| New file | Space + n | 1345 + Space |
| Save file | Space + s | 234 + Space |
| Save as file | Space + Enter + s | 2348 + Space |
| Toggle 6/8 dots | Space + p | 1234 + Space |
| Display current time | Space + t | 2345 + Space |
| Setting current folder | Space + Enter + f | 1248 + Space |

Edit Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| dot input key |  | 1-8 |
| Space |  | 9, A, 9A |
| Backspace |  | 7 |
| Enter | Joystick_Center | 8 |
| Close and exit file | Space + e | 15 + Space |
| Delete one character on the right | Space + d | 145 + Space |
| Find | Space + f | 124 + Space |
| Find Next word | Space + x, LJ_Up + RJ_Right | 1346 + Space |
| Find Previous word | Space + y, LJ_Up + RJ_Left | 13456 + Space |
| Set Mark | Space + m | 134 + Space |
| Jump to Mark | Space + j | 245 + Space |

Navigation Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Pan Left | LB |  |
| Pan Right | RB |  |
| Page up（Pan left 5 screen） | Space + LJ_Left | 13 + Space |
| Page down (Pan right 5 screen) | Space + LJ_Right | 46 + Space |
| Move to next word | LJ_Right | 5 + Space |
| Move to previous word | LJ_Left | 2 + Space |
| Cursor goes beginning of previous sentence | RJ_Up | 1 + Space |
| Cursor goes beginning of next sentence | RJ_Down | 4 + Space |
| Cursor goes beginning of current sentence |  | 14 + Space |
| Cursor goes beginning of previous paragraph | LJ_Up | 23 + Space |
| Cursor goes beginning of next paragraph | LJ_Down | 56 + Space |
| Cursor goes beginning of current paragraph |  | 2356 + Space |
| Cursor goes beginning of document | Space + LJ_Up | 123 + Space |
| Cursor goes end of document | Space + LJ_Down | 456 + Space |
| Cursor goes left | RJ_Left | 3 + Space |
| Cursor goes right | RJ_Right | 6 + Space |
| Cursor goes corresponding cell | Routing button1-16 |  |

## 2.2 File Manage

This function enables you to manage the files and folders in the Micro SD card. File Manage includes three infant items:

flm>new folder

flm>delete

flm>rename

Press Enter key or Joystick_Right in the File Manage item to go to the infant items.

2.2.1 New Folder

flm>new folder

This function enables you browse in the files tree list and press Space + n (1345 + Space) to create a new folder. You can input the new folder name, then press Enter key to create the new folder or Space + e (15 + Space) to cancel create.

Note: The folder name should not be longer than 8 characters.

2.2.2 Delete

flm>delete

This function shows the tree list and enable you delete a file or folder. You can use Joystick_Up and Joystick_Down to scroll different files, press LB or RB to display the file size, modified time and date. When you press combination key Space + d (145 + Space) in a file or folder, mini Seika will display “delete?”, press Enter to delete file, press Space + e (15 + Space) to cancel delete.

Note: If you want to delete a folder, please delete all files in this folder firstly.

2.2.3 Rename

flm>rename

Press combination key Space +r (1235 + Space), Mini will ask you input the new name of file or folder, then you can edit the file or folder name, press Enter to save the new file or folder name, if there is the same file or folder name in the current folder, mini will display “exists, replace?”, press Enter key to replace, press Space + e (15 + Space) to re-input file name. You can press Space + e (15 + Space) to cancel rename.

Note: The file and folder name should not be longer than 8 characters. The extend file name should not be longer than 3 characters. The file name and extend file name is separated by dot.

filemanage Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Create a new folder | Space+n | 1345 + Space |
| Delete file or folder | Space+d | 145 + Space |
| Rename file or folder | Space+r | 1235 + Space |

## 2.3 Read

Read function enables you read the files in u-disk or Micro SD card, and the file’s size is unlimited. In order to use the Read function, you need push a Micro SD card in the card slot or a USB disk in the USB slot. The Mini can read the brl, brf, bse, txt files and so on. If the file extend name is txt, the default display setting is 8-dots If the file extend name is brl, brf, bse or others, the default display setting is 6-dots. Mini Seika supports long file name and multi-level folder. If the file name is too long to display at one time, you can use LB and RB to display the whole file name, file size, modification date and time. When you exit the file, the mark will be added in the current place automatically.

The Read item has two infant items, sdcard and udisk. You could select one of these itmes to read the files in Micro SD card or USB-disk.

Auto scroll: The auto scroll command scrolls automatically. You can turn the auto scroll feature on by pressing Joystick_Right and stop the auto scroll by pressing Enter. For faster scrolling, press the Joystick_Right. For slower scrolling, press the Joystick_Left.

Read Commands:

| Action | Hot key | Braille |
| --- | --- | --- |
| Pan Left | LB |  |
| Pan Right | RB |  |
| Page up（Pan left 15 screen） | Space + LJ_Left | 13 + Space |
| Page down（Pan Right 15 screen） | Space + LJ_Right | 46 + Space |
| Toggle 6/8 dots | Space + p | 1234 + Space |
| Grade 2 Braille Toggle(only for English txt document) | Space + g | 1245 + Space |
| Escape | Space + e | 15 + Space |
| Begin auto scroll | Joystick_Right |  |
| Stop auto scroll | Enter |  |
| Increase auto scroll speed | Joystick_Right |  |
| Decrease auto scroll speed | Joystick_Left |  |

## 2.4 USB connect mode

This application will enable you to use your mini Seika as a PC input/output interface. You will need to connect the USB cable and to launch the screen reader on the PC. Mini Seika support JAWS, Window-eyes, and HAL.

Once this application is started, Mini will display “wait connect…”. Then you can start Screen Reading Software.

## 2.5 Bluetooth mode

Using this application will allow you to use the Bluetooth interface of Mini Seika. Once this application is started, Mini will display “wait connect...”, 2 second later will display “btname>tsm ****”, where **** is the Bluetooth name serial number which is for division when there are a couple of Mini in one room.

You could input “0000” when you are asked to input the Mini Seika Bluetooth pin code to set up the connection.

## 2.6 Clock

This application can display and set the time and date. There are five infant items:

2.6.1 Display Current Time

For example, if the time is 16:20:12, mini Seika will display cl>16h20m12s in 24 hours system, cl>04h20m12s pm in 12 hours system. You could press Enter or Space + e (15 + Space) to escape.

2.6.2 Display Current Date

For example, if the date is 25.06.2011, Saturday, mini Seika will display cl>sat 25d06m11y. You could press Enter or Space + e (15 + Space) to escape.

2.6.3 Set Time

cl>set time

Press Enter key to begin time setting by changing the following number: hxx mxx sxx am. Press Joystick_Left and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space + e (15 + Space) to cancel setting.

2.6.4 Set Date

cl>set date

Press Enter key to begin date setting by changing the following number: dxx mxx yxx. Press Joystick_Left and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space + e (15 + Space) to cancel setting.

2.6.5 Set hour format

cl>time format

Press Enter key to begin time format, the mini will display “12 24”. You can select 12 or 24 hours format by Press Joystick_Left and Joystick_Right to move the routing cursor. You will need to press Enter to accept the new setting or press Space + e (15 + Space) to cancel setting.

The time format is 12 hours in default.

## 2.7 Calculator mode

With calculator you can execute arithmetical function as addition, subtraction, multiplication, division.

| Operator | Standard Braille | Braille |
| --- | --- | --- |
| Decimal separator | . | 46 |
| Addition | + | 346 |
| Subtraction | - | 36 |
| Multiplication | * | 16 |
| Division | / | 34 |
| Equal | = or Enter | 123456 or 8 or Joystick_Center |
| Clear | Backspace | 7 |

## 2.8 Tools mode

With tools application you will be able to access some useful information concerning your device. You can enter Tools infant items by press Enter or Joystick_Right.

2.8.1 Power Menu

tls>power

It shows you the power source and battery capacity. If your Mini is not plugged, the Mini will display “not charged xx%”, when the voltage is too low (Below 3.65V), the Mini will beep three times Per 10 minutes, and it will display “please charge”. If you plug in the adapter, then Mini Seika will beep twice and display “usb charging xx%”. When the battery is charged fully, the Mini will beep three times alarm, and it will display “bat 100%”.

You could press Space + e (15 + Space) or Enter key or Joystick_Left to exit.

2.8.2 Update

tls>update

This function allows you to update the firmware. You need download the update file MINI****.BIN into USB disk, and plug it into Mini Seika USB slot. Press Enter key, mini Seika will update the firmware. It will take about 8 seconds. After update complete, it will beep twice and display “pls restart”, then close Mini and restart.

Note 1: You should not plug out USB disk or turn power off when updating the firmware. Otherwise, the Mini firmware will be broken.

Note 2: If the update fail, and the Mini Seika can’t restart, don't worry! Keep pressing Space + u (136 + Space) and turn power on, the Mini Seika will display “update”, then press Enter key, it will start update.

2.8.3 Test

tls>test

This function allows you to test Mini Seika cell pins and keys. There are two infant items:

ts>braille line

This function will move all pins in sequence. Press Enter key to change the display test mode. Press Space + e (15 + Space) or Joystick_Left to escape.

ts> keyboard

This function will enable you to test each key in Mini. When you press any key and release it, the display pins will change and Mini will beep once. Press Space + e (15 + Space) or Joystick_Left to escape.

2.8.4 About

tls>about

This feature display the current version of the program, the format is “*.***”, such as “v1.001”. Press Enter or Space + e (15 + Space) or Joystick_Left to escape.

2.8.5 Set sleep

tls>set sleep

Press Enter key or Joystick_Right to begin sleep time setting, the mini will display “1 minute”. You can select sleep time by press Joystick_Up and Joystick_Down, such as select 5 minutes, then press Enter to accept the new setting or press Space + e (15 + Space) or Joystick_Left to cancel setting.

The sleep time is 1 minute in default. You can set sleep time between 1 minute to 59 minute.

2.8.6 Help

tls>help

This function will show the help document in the Micro SD card. You could use Read function hot key to read the help document. Press Space + e (15 + Space) to escape.

# 3. Configuring the Bluetooth Connection

This section shows you how to install and setup the Bluetooth in your PC. If there is Bluetooth device in your computer, please skip section 3.1 and go to section 3.2. If there is no Bluetooth device in your computer, please follow the steps in section 3.1.

## 3.1 Install the Bluetooth dongle driver

Open the “USB Bluetooth dongle Driver” folder in the CD. Run the “setup.exe”.

Install the driver by default steps. The Bluetooth dongle Driver will be installed automatically. A console window appears and it will take several minutes.

Click “finish”, the Bluetooth dongle Driver is installed successfully.

You could find an USB Bluetooth dongle in the Mini package and please plug it in your computer.

## 3.2 Setup the Bluetooth Connection

The Mini Seika could connect with most of Bluetooth devices. These Bluetooth devices could be embedded in the laptop or mobile phone, or a USB dongle plug in the PC.

Bluetooth is a short-range wireless communications technology. Mini Seika with Bluetooth capability can communicate with computer over a distance of about 10 meters (30 feet) without requiring a USB cable.

In order to use Mini Seika through Bluetooth, you need establish a partnership with a Bluetooth enabled computer. Most of laptops offer built-in Bluetooth. If there is no built-in Bluetooth in your computer, you need plug in an external USB Bluetooth dongle (you could find it in the Mini Seika package) and install the appropriate drivers on your computer.

Here are instructions for setting up the Bluetooth connection between the Mini Seika and computer.

Do not connect USB cable with PC (you could use USB cable and power adapter for charging the Mini).

Turn on the Mini Seika and open Mini Bluetooth following the steps in section 2.5.

Make sure that Bluetooth is enabled on your computer. If you are using an external USB Bluetooth adapter, make sure insert it into your computer.

Double click the Bluetooth icon in the Windows desktop right –down corner to open Bluetooth control software, and search for Bluetooth devices. It will take several seconds. When the search is complete, the Braille display Bluetooth name TSM is in the list of devices.

Select TSM from the list. If the computer doesn’t find the Mini Seika, please make sure you turn on the Mini Seika, and then search again.

In the Bluetooth pin passkey edit field, type Mini Seika pin code “0000”.

After a few seconds, you will receive a message that the Bluetooth device was successfully connected and the outgoing and incoming COM port assignments are displayed. Make a note of the outgoing port number as this is the port the screen reader will use to communicate with the Mini Seika.

Now please configure your screen reader to use the Bluetooth COM port. To configure the COM port in the screen reader, please refer to the chapter 4 for Jaws, chapter 5 for Window-eyes.

# 4. Using Mini Seika with JAWS

## 4.1 Installing Seika Patch for JAWS

Open the “patch for JAWS” folder in the CD. Run the Patch for JAWS according to your Windows operating system.

Install the patch by default steps. The Patch for JAWS will be installed by the JAWS Utility automatically. A console window appears and it will take several minutes.

## 4.2 Working in JAWS

### 4.2.1 Configuring JAWS

Start JAWS and select “Braille…” in the “Options” menu.

Assuming that the default Braille display is set to “No display”, tab to the “Add Braille Display” button and press enter.

In the “Select components” dialog box, there is a list of Braille displays. Select the “Seika” by pressing the space bar.

Tab to the “Next” button and press enter.  If you connected Mini Seika with your computer by USB cable, select “USB” as the output port.

If you connect Mini Seika to PC using Bluetooth, in the “Set device options” dialog box, select the output port that corresponds to the COM port number that you make the note in chapter 3.2.Tab to the “Next” button and press enter.

When prompted to set the primary device, select “Seika” in the “Primary device” list.

Tab to the “Finish” button and press enter.

In the “Braille Basic Settings” dialog box, tab to the “OK” button and press it.

Restart the JAWS and the Mini Seika will work now.

### 4.2.2 Navigation and control keys function

| Action | Keys |
| --- | --- |
| Enter | 9A |
| Enter | RJ_Center |
| Space | A |
| JAWS Backspace | 9 |
| Up Arrow Key | RJ_Up |
| Down Arrow Key | RJ_Down |
| Left Arrow Key | RJ_Left |
| Right Arrow Key | RJ_Right |
| Braille Pan Right | RB |
| Braille Pan Left | LB |
| Stop Speech | 9+RB |
| Braille Moves Active | LJ_Left + RJ_Up |
| Active Moves Braille | LJ_Left + RJ_Down |
| Braille Toggle Marking | LJ_Center + RJ_Up |
| Braille Next Line | LJ_Right |
| Braille Prior Line | LJ_Left |
| Start JAWS Task List | LJ_Right + RJ_Up |
| List Task Tray Icons | LJ_Right + RJ_Dowm |
| Right Mouse Button | LJ_Right + RJ_Right |
| Left Mouse Button | LJ_Right + RJ_Left |
| Left Mouse Button Lock | A+LJ_Right |
| Route JAWS Cursor To Pc | LJ_Right + RJ_Center |
| Route PC Cursor To JAWS | LJ_Left + RJ_Center |
| Braille Bottom | LJ_Down |
| Braille Top | LJ_Up |
| Minimize All Apps | 9 + LJ_Center |
| Select A Link | LJ_Down + RJ_Left |
| Select A Frame | LJ_Up_Up + RJ_Down |
| Select A Heading | LJ_Up + RJ_Left |
| Select A Form Field | LJ_Up + RJ_Right |
| Braille Toggle 8 Dots | A + LB |
| Bottom Of File | LJ_Up + RJ_Up |
| Top Of File | LJ_Down + RJ_Down |
| PC Cursor | RB + LJ_Right |
| JAWS Cursor | RB + LJ_Left |
| Braille Toggle Mode | LB + RB |
| Braille Toggle 8 Pixels Per Space | A + RB |
| Braille Toggle Characters And Attributes | LJ_Left + RJ_Left |
| Braille Turn Marking Off | 9 + LB |
| Increase Braille Auto Advance Speed | LB + RJ_Center |
| Decrease Braille Auto Advance Speed | LB + LJ_Center |
| JAWS Page Down | LJ_Down + RJ_Center |
| JAWS Page Up | LJ_Up + RJ_Center |
| Braille Tab | LJ_Center + RJ_Right |
| Braille Shift Tab | LJ_Center + RJ_Left |
| Next Document Window | RB + LJ_Down |
| Previous Document Window | RB + LJ_Up |
| Route Braille To Active Cursor | RB + LJ_Center |
| Braille Select Text | RJ_Center + Routing |
| Repeat Last Flash Message | LJ_Down + RJ_Up |
| Braille Auto Advance | LJ_Down + RJ_Right |

## 4.3 Uninstalling the patch of JAWS

Note: Uninstall the patch of JAWS will disable the Mini Seika work with JAWS and delete all the patch files.

You must uninstall this patch before uninstall the JAWS.

### 4.3.1 Uninstall the old version of Seika patch for Jaws

Open the “Add or Remove Programs” in the “Control Panel”, and select the item: “Seika Braille Display version 3” or “Seika Patch for JAWS V1.x” or “Seika Patch for JAWS V2.x”. Uninstall the item step by step. The uninstall will taking several minutes. Please restart computer after finished.

### 4.3.2 Uninstall the current version of Seika patch for Jaws

Open the “Add or Remove Programs” in the “Control Panel”, and select the item: “Seika Patch for JAWS V2.3”. Uninstall the “Seika Patch for JAWS V2.3” step by step. The uninstall will taking several minutes. Please restart computer after finished.

# 5. Using Mini Seika with Window-Eyes

## 5.1 Configuring Window-Eyes 7.5.2

Note: We strongly recommend you update the Window-Eyes to version 7.5.2 or later.

If you connect Mini Seika to your computer using Bluetooth, in the “Set device options” dialog box, select the output port that corresponds to the COM port number that you make the note in chapter 3.

Start Window-eyes 7.5 and select “Device” in the “Settings” list.

Select “Braille Display” in the “Device” list.

In “Braille Displays” option, select “Seika” option.

In “Ports” option, if you connect Mini Seika to your computer using USB, select the USB connect.

If you connect Mini Seika to your computer using Bluetooth, select the Seika COM port number that you make the note in chapter 3.

Tab to the “Activate” button and press Enter.

Select “Keep Changes” button in the “Braille Settings” dialog box, then press Enter.

Tab to “Save Settings” button and press Enter.

## 5.2 Navigation and Control keys function

| Actions | Keys |
| --- | --- |
| Enter | 9A |
| Enter | RJ_Center |
| Space | A |
| Backspace | 9 |
| Scroll Braille Left | LB |
| Scroll Braille Right | RB |
| Next Braille Line | LJ_down |
| Next Braille Line | RJ_down |
| Prior Braille Line | LJ_up |
| Prior Braille Line | RJ_up |
| Top Line | LJ_Left |
| Top Line | RJ_Left |
| Bottom Line | LJ_Right |
| Bottom Line | RJ_Right |
| Mouse Bottom Left | LB + RJ_Down |
| Mouse Bottom Right | RB + LJ_Right |
| Mouse Top Left | LB + RJ_Up |
| Mouse Top Right | RB + LJ_Left |
| To Focus | LB + RB |
| 6/8 Dot Toggle | Space + LB |
| Untranslate at Cursor | LJ_Center + RJ_Left |
| Untranslate at Mouse | LJ_Center + RJ_Right |
| Braille Graphic Mode | LJ_Center + RJ_Down |
| Scroll Mode Toggle | LJ_Up + RJ_Left |
| Show Attributes Toggle | LJ_Down + RJ_Center |
| Grade 2 Braille Toggle | LJ_Center + RJ_Center |
| Braille Tracking Toggle | LJ_Down + RJ_Down |
| Spacing Toggle | LJ_Down + RJ_Right |
| Attributes To Show Rotor | LJ_Up + RJ_Center |
| Display Individual Attribute Rotor | LJ_Center + RJ_Up |
| Quick Message Rotor | LJ_Down + RJ_Left |

# 6. Using Mini Seika with VoiceOver

## 6.1 Using VoiceOver on iPad and iPhone

Do not connect USB cable with PC (you could use USB cable and power adapter for charging the Mini).

Turn on the Mini Seika and open Mini Bluetooth following the steps in section 2.5.

Press Home key on iPad or iPhone.

Tap Settings. Then double-tap Settings.

Tap General. Then double-tap General.

Use finger on the screen to scroll to Accessibility.

Tap Accessibility. Then double-tap Accessibility.

Tap VoiceOver. Then double-tap VoiceOver.

Turn VoiceOver is on.

Use three fingers on the screen to scroll to Braille.

Tap Braille.

The Braille display Bluetooth name will appear in the list of Braille devices. Its name will begin with TSM in the list of discovered devices.

Single-tap the device name to select it, double-tap the device name to initiate the pairing.

You will be taken to a passcode entry screen, enter the passcode 0000.

## 6.2 Using VoiceOver on a Mac OS X computer

Do not connect USB cable with PC (you could use USB cable and power adapter for charging the Mini).

Turn on the Mini Seika and open Mini Bluetooth following the steps in section 2.5.

Launch VoiceOver Utility (/Applications/Utilities/VoiceOver Utility.app)

Click on the "Braille" tab on the left.

Click on the "Displays" tab.

The Braille display Bluetooth name will appear in the list of Braille devices. Its name will begin with TSM in the list of discovered devices.

Click the device name to select it, and initiate the pairing.

You will be taken to a passcode entry screen, enter the passcode 0000.

## 6.3 Navigation and control keys function

Note:When using a Braille display with VoiceOver using iOS 6 or later, you can use the following key commands to navigate.

Common Braille commands for VoiceOver navigation:

| VoiceOver action | Braille command (chord) |
| --- | --- |
| Move to previous item | Dot 1 + Space bar |
| Move to next item | Dot 4 + Space bar |
| Move to previous item using rotor setting | Dot 3 + Space bar |
| Move to next item using rotor setting | Dot 6 + Space bar |
| Select previous rotor setting | Dot 2 + Dot 3 + Space bar |
| Select next rotor setting | Dot 5 + Dot 6 + Space bar |
| Move to the first element | Dot 1 + Dot 2 + Dot 3 + Space bar |
| Move to the last element | Dot 4 + Dot 5 + Dot 6 + Space bar |
| Read page starting at selected item | Dot 1 + Dot 2 + Dot 3 + Dot 5 + Space bar |
| Read page starting at the top | Dot 2 + Dot 4 + Dot 5 + Dot 6 + Space bar |
| Activates the Home button | Dot 1 + Dot 2 + Dot 5 + Space bar |
| Goes to the Status bar | Dot 2 + Dot 3 + Dot 4 + Space bar |
| Activates the Volume Up button | Dot 3 + Dot 4 + Dot 5 + Space bar |
| Activates the Volume Down button | Dot 1 + Dot 2 + Dot 6 + Space bar |
| Activates a Back button if present | Dot 1 + Dot 2 + Space bar |
| Activates the Eject key | Dot 3 + Dot 4 + Space bar |
| Activates the Delete key | Dot 7 + Space bar |
| Activates the Delete key | Dot 1 + Dot 4 + Dot 5 + Space bar |
| Activates the Return key | Dot 8 + Space bar |
| Activates the Return key | Dot 1 + Dot 5 + Space bar |
| Activates the Tab key | Dot 2 + Dot 3 + Dot 4 + Dot 5 + Space bar |
| Toggle Screen Curtain on and off | Dot 1 + Dot 2 + Dot 3 + Dot 4 + Dot 5 + Dot 6 + Space bar |
| Pause or Continue speech | Dot 1 + Dot 2 + Dot 3 + Dote 4 + Space bar |
| Toggle speech on and off | Dot 1 + Dot 3 + Dot 4 + Space bar |
| Scroll left one page | Dot 2 + Dot 4 + Dot 6 + Space bar |
| Scroll right one page | Dot 1 + Dot 3 + Dot 5 + Space bar |
| Deselect text | Dot 2 + Dot 3 + Dot 6 + Space bar |
| Select text | Dot 3 + Dot 5 + Dot 6 + Space bar |
| Pan Braille to the left | Dot 2 + Space bar |
| Pan Braille to the right | Dot 5 + Space bar |
| Toggle announcement history | Dot 1 + Dot 3 + Dot 4 + Dot 5 + Space bar |
| Switch between contracted and uncontracted Braille | Dot 1 + Dot 2 + Dot 4 + Dot 5 + Space bar |

Device keys for VoiceOver navigation:

| Voice action | Braille device key |
| --- | --- |
| Pan braille to the left | LB |
| Pan braille to the right | RB |
| Select item under your finger | LJ_Center |
| Move to previous item | LJ_Left |
| Move to next item | LJ_Right |
| Move to previous item using rotor setting | LJ_Up |
| Move to next item using rotor setting | LJ_Down |
| Select item under your finger | RJ_Center |
| Move to previous item | RJ_Left |
| Move to next item | RJ_Right |
| Move to previous item using rotor setting | RJ_Up |
| Move to next item using rotor setting | RJ_Down |
| Double tap the selected item | Router |
| Toggle expanded status description | Status Router |

1.2 Device description	2

1.3 Power Switch and Battery	5

1.4 Hardware Specifications	6

2.0.1 How to Use the Commands	7

2 Basic Functions of the Mini Seika	7

2.0.2 General Functions	7

2.1 Notepad	9

2.2 File Manage	16

2.3 Read	18

2.4 USB connect mode	20

2.5 Bluetooth mode	20

2.6 Clock	20

2.7 Calculator mode	22

2.8 Tools mode	22

3.1 Install the Bluetooth dongle driver	26

3. Configuring the Bluetooth Connection	26

3.2 Setup the Bluetooth Connection	26

4.1 Installing Seika Patch for JAWS	29

4. Using Mini Seika with JAWS	29

4.2 Working in JAWS	29

4.2.1 Configuring JAWS	29

4.2.2 Navigation and control keys function	30

4.3 Uninstalling the patch of JAWS	33

4.3.1 Uninstall the old version of Seika patch for Jaws	33

4.3.2 Uninstall the current version of Seika patch for Jaws	33

5.1 Configuring Window-Eyes 7.5.2	34

5. Using Mini Seika with Window-Eyes	34

5.2 Navigation and Control keys function	35

6.1 Using VoiceOver on a ipad	37

6. Using Mini Seika with VoiceOver	37

6.2 Using VoiceOver on a Mac OS X computer	38

6.3 Navigation and control keys function	38